AWS2821: PRINTER MANAGER

========================

AWS2821 emulates a mainframe line printer on a PC printer (or to an OS/2 file).

DEVICE TYPE: 1403, 3211

============

Manager Parameters

==================

Input parameter (optional):

time delay    = Time delay in seconds.  If no printing activity occurs during

this period the print file is closed.  Default is 10 seconds.

This parameter can be set using the P/390 Configurator

"Update System Devices" menu "Device Manager Parm

Edit/Display" submenu (F2/F12).

e.g. a parameter of "60" would wait 1 minute before closing files.      */

Configuring a printer

=====================

To configure a printer device go to the Configurator F2 "UPDATE SYSTEM

DEVICES" menu and enter the real address for the 1403 or 3211 printer in the

Addr field, the MGR code for AWS2821 in the Mgr field, and the following

optional parameters in the FN/P field:

Either:

------

filename       Fully qualified file name for printing to a file.

Default is to print to the PC printer LPT1.

LPTn (no colon) is also allowed.

The file is opened in append mode.

/80            Print in 80 column mode. Default is 132 columns.

Records are truncated to 80 columns.  80 column mode also

sets a larger font, 132 column mode sets a smaller one.

/I             Send a printer initialization string to an IBM 40x9 Laser

Printer to switch it to PPDS mode. Default is NOT to send

any initialization string.

Or:

---

/C=filename    Points to an ASCII file that contains options for advanced

printer settings. When using this file, it replaces all of

the above options, so DO NOT specify any of them.

All these options can be specified in the control file

along with the new functions explained below.

Examples:

LPT2

LPT1 /80

G:\PRINTER\output.00E

D:\mydata\script.prt /80

LPT2 /I

/C=C:\P390\PRINT.CTL

The following is an example of a control file. You can use any normal text

editor to build the file. Comments are preceded by "//".

// This file is used by a printer defined in devmap for special printer

// control sequences. It also replaces the normal parameters that would

// have been in the "parm" field of the devmap entry. If this file is not

// specified, those parameters can still be used.

// The entries are defined as follows (case is ignored except for EOJ):

// NOTE! You can type up to 150 CHARACTERS per line in any of the following

//       entries. Some of the commands can read multiple lines. Those that

//       support multiple lines are indicated below.

// ----------------------------------

//

// PC   is a string of hex values that will be sent to the printer to place it

// in a particular mode. The user must consult the printer reference manual

// to find the proper codes. If invalid codes are used, unpredictable results

// may occur. The printer manager in the P/390 system will NOT check for

// the proper codes. Each number MUST be a 2 character pair of hex values.

// Example: PC=ab 3e f1 11 22 33 01 10

//

// This command supports MULTIPLE LINES. You can specify multiple PC=

// statements, one per line. As each PC= line is read, the data is ADDED

// to the previous PC= line. The MAXIMUM allowed number of control chars

// (from all PC= lines) is 255. By using multiple lines, you can group

// the control sequences so that each function could be on its own line.

// This makes it easier to debug and maintain the control sequences.

// The blanks that you use to delimit the characters are removed when the

// line is read, so don't count the blanks in the final total of characters

// in the control sequence.

// --------------------------------------------------------------------------

// EOJ   is a string to search for to indicate the end of a print job. The

// first number is the line number, the second number is an offset into the

// line, and the string to search for is in single quotes. Blanks are allowed.

// If 0,0 is coded for the row and column, only the string is searched and

// the row and column positions will be ignored.

// Example: EOJ=11,25,'this is the end'

// --------------------------------------------------------------------------

// LL= n   where n is a number for the maximum printed output length of any

// line. Lines longer than the max will be truncated. The default is 132.

// --------------------------------------------------------------------------

// FILE= name   where name is the name of a file where the printed output will

// be placed instead of actually being printed. This file can then be printed

// later as all of the control characters and data will be in the file.

// NOTE! This command differs slightly from the version that can appear in

//       the parameter field in the DEVICES frame of the Configurator. In

//       that case only the file name or printer designation is specified.

//       In this control file "FILE=" must appear before the file name or

//       printer designation.

// Examples:   file=lpt1

//             file=c:\printer\print.data

// --------------------------------------------------------------------------

// LINE NUMBERS   This string activates a tool to help you in coding for the

// offsets when using the EOJ string. It will place ### nnn  ### into the

// first 20 characters of each printed line so that you can see the actual

// line number of the printed lines. This also turns off the EOJ search if

// activated. When you have found the line number you wish to use, then

// comment out or remove this line from the file.

// --------------------------------------------------------------------------

// TR   This allows the user to change specific entries in the EBCDIC to

// ASCII translate table. The syntax is fairly strict:

//    TR=CX1 CX2 CX3,NX1 NX2 NX3

// Each code point is represented by a 2 character hex value.  CX is the

// CURRENT EBCDIC HEX value to be translated, NX is its corresponding NEW ASCII

// replacement value. All "standard" codes are on the left, all new codes are

// on the right side of the comma.

// There must also be the same number of hex values on the left as on the right

// Case does not matter and blanks separating the values are optional.

//

// This command supports MULTIPLE LINES. You can specify multiple TR=

// statements in the control file and they can be in any order. Since this

// command REPLACES a translate table entry based on the hex index using the

// OLD value on the left of the comma, this command is NOT additive. This

// simply means that you can replace all 256 entries in the default translate

// table using as many TR= statements as you wish. Duplicate entries do not

// cause any problems, however, the last code point found is the one that will

// be used.

//

// --------------------------------------------------------------------------

// Some sample entries follow (DON'T use these without modifying them):

// Put IBM Laser Printer in HP Emulation mode, Courier 16.67 (small font),

//  landscape mode, 12 lines/inch:

pc=1b 5b 4b 05 00 05 31 02 00 0a 1b 26 6c 31 4f 1b 26 6c 31 32 44

// Same as above, but with multiple lines:

pc=1b 5b 4b 05 00 05 31 02 00 0a

pc=1b 26 6c 31 4f

pc=1b 26 6c 31 32 44

// Specify End Of Job string:

eoj=54,5,'EEEE'

// Set line length to 132 columns:

ll=132

// Redirect printer output to a file:

file=d:\printer.out

// Turn on line numbers (for debugging ONLY):

line numbers

// Translate 'A B C' to 'a b c':

tr=c1 C2C3,61 62 63

// Same as above, but with multiple lines:

tr=c1,61

tr=c2,62

tr=c3,63

----------------------------------------------------------------------------

When working with the EOJ feature, it is recommended that the user try

a small test job first. If the EOJ characters cannot be found, the job will

never print. In that case, you must unload the AWS2821 print manager and

then reload it. This will flush the job from the system and you can try again.

The control file is only read when AWS2821.EXE is loaded. If you change the

control file you must first unload the printer manager. Type in an OS/2 window:

awsstat aws2821

Then reload it to pick up the control file changes:

cd \p390

awsstart aws2821.exe n