AWS5080: 5080 DRIVER

====================

AWS5080 provides 5088-like function using FSLA or MSLA adapter cards.

DEVICE TYPE: HFGD 3278, 3279

============

CONFIGURING A 5080

==================

To configure a 5080, go to the configurator UPDATE SYSTEM DEVICES (F2) screen

and enter a real device address in the Addr field and a manager code in the Mgr

field.  Leave the Filename/Parameters field blank.

Devices defined will form two polling ranges, one for graphics devices and one

for 3270 devices.  The ranges must be disjoint; there can be gaps within a range

of like devices but there can be no mixing of the two device types within a

given range.  Each device range should be no larger than necessary (for optimal

polling performance) and cannot exceed 16 devices.  The two ranges should be

defined to the S/390 operating system as two independent control units (e.g.,

for VM, the graphics range will be defined as an HFCU control unit and the 3270

range will be defined as a 3274 control unit).

Leave the comment field blank.