AWS2540: Card Reader Emulator

=============================

AWS2540 emulates a card reader.  It watches for files to appear in an OS/2

subdirectory.  Files copied into that subdirectory appear to the S/390 as card

decks placed into a stacker.  The card reader emulator makes it easy for

PC LAN clients to submit jobs to a P/390 VM or VSE operating system.

DEVICE TYPE: 2540

===========

Configuring a 2540 card reader

==============================

Only one 2540 is permitted in DEVMAP.  Go to the Configurator F2 "UPDATE

SYSTEM DEVICES" menu and add a 2540 device at an appropriate address in your

I/O configuration and specify the MGR code for AWS2540.  In the FN/P field,

put the fully qualified filename.  You must have at least one "*" in the name,

and you must specify a subdirectory (i.e. you need at least two "\" characters

in the name).  The directory can be located on a local drive, or on a file

server.  Here is an example entry:

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>00C   >2540                         >7   >G:\RDR\*.JCL

Make sure that you select an address where the operating system expects to see

a card READER (in VM, 2540R).  Do not confuse this with the card PUNCH, which

also has the device type 2540 (in VM, 2540P).

Optional Parameter:  /NT

==================

This parameter is useful for VSE when you are NOT running POWER.

Put this parameter after the file name in the FN/P field to disable all

time-outs.  If you do not specify /NT then AWS2540 will remove a card deck if

the S/390 doesn't read at least once a minute (after having read the first card).

See "Saving BAD card decks", below.

Saving "OLD" card decks

=======================

The 2540 manager watches the directory specified in DEVMAP for new files and

reads them one at a time.  Once a file is read it is normally erased.  If you

want the 2540 to save the "old" files it has read, create a subdirectory under

the one specified in devmap.  Call the new subdirector 'OLD'.  In the example

above, you would use the OS/2 MKDIR command to create a subdirectory

G:\RDR\OLD.  Each time a file is completely read, it is moved to the OLD

subdirectory if it exists.  If the OLD subdirectory doesn't exist, the file is

erased.  If a file with the same name as one just read already exists in the

OLD subdirectory, it is replaced.

Saving "BAD" card decks

=======================

When VM finds an invalid card deck in the reader, it stops reading, issues a

message and waits the for operator to remove the card deck and put in a new

one.  AWS2540 automatically removes a card deck if the S/390 doesn't read a card

at least once a minute (after having read the first card).  These "BAD" card

decks are either erased, or moved to a subdirectory named "BAD".  Using the

example above, AWS2540 would try to move the file to G:\RDR\BAD.  If the BAD

subdirectory does not exist, the file is erased.

ASCII/EBCDIC translation and BINARY card decks

==============================================

When the 2540 reads the first card of a card deck, it checks the first few

characters to see if they are "ID", "//", or "USERID".  It makes this check in

ASCII.  If any of these match, ASCII/EBCIDIC translation is assumed to be

required for the whole file.  Each ascii "line" is delimited by a "end of line"

or X'0D0A' sequence.  Short lines are padded to 80 bytes with blanks.  Long

lines are split into multiple 'cards'.

If the first characters are not "ID", "//", or "USERID", then it is assumed

that the file is binary and doesn't need translation.  The file is read 80

bytes at a time until exhausted.

Order of reading

================

AWS2540 tries to read the oldest decks in the directory first.  There are

several time and date stamps associated with an OS/2 file:  Created, last

written, last accessed, etc.  AWS2540 uses the 'last written' date.  The OS/2

file system only keeps timestamps to a 2 second resolution, so files written to

the directory within a 2 second intervale may be read out of order.

Note: the OS/2 copy command doesn't update the 'last written' date when it

copies a file unless specifcally tell it to.  Use the "+ ,," option to copy a

file to a directory with a new date.

For example:  CD G:\RDR

COPY H:\JOBLIB\REPORT4.JCL + ,,

Will copy the report4.jcl job into G:\RDR with the current date and time.

ERROR MESSAGES

===============

AWSRPU002E Invalid path for $x2.  Path=$s.

Explanation: You must specify a subdirectory on the filename/paramater

field.  The path must contain at least 2 "\" and at least one '*'.

AWSRPU099E Too many AWS2540 devices, only one unit is supported.

Explanation: More than one 2540 device was found in DEVMAP.  Only the

first is used; the rest are ignored.