AWSTFA - TRANSPARENT FILE ACCESS

================================

TFA lets you link and access Host VM system CMS minidisks directly

on P/390 workstation CMS.

DEVICE TYPE: 3088

============

CONFIGURING TFA DEVICES

=======================

To configure a TFA device, go to the P/390 Configuration "Update System Devices"

(F2) screen, and enter a real address in the Addr field, a manager code in the

Mgr field, and the Server name (AWSHOST) or Server alias (AWSHOST1)

in the Filename/Parameters field.

o A Server alias is used for CM/2 (version 1.11)

o The Server name is used by PC/3270 (version 4.1 or above)

The Server alias is defined in the OS/2 Communications Manager

Server-Requester Programming Interface (SRPI) profile.  The usual value

for Server alias is AWSHOST1 for the first host VM session, and AWSHOST2

for the second host VM session. Only a single SRPI session is supported by

PC/3270.

PROGRAM REQUIREMENTS

====================

To enable TFA in your workstation, three groups of programs are involved:

1. Host VM system programs:

- CMS command CMSSERV

- AWSHOST MODULE (the P/390 host Server program)

- AWSHOST SRPI (loads the Server program)

2. OS/2 PC programs:

- CM/2 or PC/3270 SRPI

- AWSTFA.EXE (P/390 TFA I/O device manager)

3. Workstation CMS programs:

- TFA EXEC

- TFAESA MODULE (P/390 TFA nucleus extension)

- HLINK command

- HDETACH command

- HQUERY command

You are responsible for the following steps:

1. Getting the TFA host programs on your host system.

2. Installing and configuring Communications Manager/2 or PC/3270 SRPIsupport

Configuring the TFA device(s) via the P/390 Configuration program

3. CMS TFA comes with the P/390 Starter System CD-ROMs

These steps are covered in the README.AWS file which is shipped with the

P/390 Program diskette.

A review of the P/390 Configuration steps follows:

- Start the P/390 Configuration program (AWSCFG.CMD)

- Select "F2 Update System Devices"

- Scroll to device 450

- Make sure "Mgr" code corresponds to MGR Code=TFAMGR

(If not, delete the device and add it again with the correct mgr number).

- In the "Filename/Parameters" field on the right for device 450,

o For CM/2, type AWSHOST1 and press ENTER.

o For PC/3270, type AWSHOST and press ENTER.

- If you have a second host session and wish to do TFA to that

session, add a TFA device at address 451 and type AWSHOST2 in the

Filename/Parameters field.  Press ENTER.

*** Only one TFA host session is supported at a time using PC/3270 ***

However, device 450 just defined can be used from ANY session.

- Press F6 to save the configuration.

or

- Press F10 or ESC to return to FUNCTIONS menu.