LAN3088: 3088 Emulation over NetBios (Token/ring) LAN

======================================================

The 3088 emulator lets you run programs like RSCS, PVM, TSAF and the shared file

system over a LAN between P/390 systems running VM.  It is used only for

connecting P/390 systems.  It does not allow connections to 9370 systems or

mainframes through a 3172.

DEVICE TYPE: CTC or 3088

===========

CONFIGURING A LAN3088

=====================

To configure a LAN3088, go to the P/390 Configuration "Update System Devices" (F2)

screen and for each LAN3088, enter a real address in the Addr field and a

manager code in the Mgr field.  Use the Filename/Parameters field to specify the

connections.  Specify connections in the form 'node.appl', where node is the

target nodeid, and appl is the application name.  The nodeid for a P/390 is

specified on the F4 frame; it can be up to 8 characters long.  The name is 4

characters long, and is used to connect two 3088's symbolically.  It can be

anything you like, but is normally the name of the S/390 application to which

you want to attach this 3088.

NOTE: Do NOT configure more than TEN 3088 or CTC device addresses.

Example:

In this example, there are 3 P/390 systems on the same LAN.  PVM connections run

between system 1 and system2.  Net (rscs) connections are established between

systems 1&2, 2&3, and 1&3.  TSAF connections are present between systems 1&2,

and between systems 2&3.

Device address        Sys1             Sys2           Sys3

440              Sys2.pvm  <--->  Sys1.pvm    +-> sys1.net

+----------------+

441              sys3.net  <-+    sys3.net  <---> sys2.net

442              Sys2.tsaf <--->  sys1.tsaf   +-> sys2.tsaf

|

443                               sys3.tsaf <-+

Note the following:  (1) The names are case insensitive.  (2) The device address

on different systems don't have to be the same on for a connection; connections

are established via the 'appl' parameter.

By convention, full duplex links as used in the PVM should have names that end

in "e" or "o" for even and odd addresses.  That helps insure that you get them

connected to one another and attached to the virtual machine correctly.

Device address        Sys1             Sys2           Sys3

440              Sys2.pvme <--->  Sys1.pvme

441              sys2.pvmo <--->  sys1.pvmo

442              Sys3.pvme <--------------------> sys1.pvme

443              Sys3.pvmo <--------------------> sys1.pvmo

Error Messages

--------------

The most common error messages is CTC400, which indicates that there were not

enough LAN resources to satisfy all the node/applications that you specified in

the P/390 Configuration:

CTC400E RESET SHORT (xrc) REQ (s,names,ncbs) GOT (s,names,ncbs)

xrc   = is the return code in hex,

s     = the number of sessions requested (and gotten)

names = the number of names requested (and gotten)

ncbs  = the number of network control blocks requested (and gotten)

If you get this message, there are three possible solutions:

(1) reduce the number of LAN3088 device you have defined.

(2) reduce or disable other LAN applications you have -- for example,

LAN3274 sessions, server sessions, etc.

(3) Increase your LAN parameters in the communications manager.

The README file that comes with the P/390 gives some information

on how to do this.

AWS3088 - DEVMAP PARAMETERS

����������������������������������������������������������������������������ͻ

�Type  Addr  Label  Atype  Size     Mgr  Filename/Parameters  Disk:>C<       �

�-----���Ŀ------------------------�Ŀ-������������Ŀ------------------------�

�     �602�>       >     >         �8� �RALVMPA.NETe�                        �

�3420 �����                        ��� ��������������                        �

�3088  600                         >8   RALVMPA.PVMe                         �

�3088  601                         >8   RALVMPA.PVMo                         �

����������������������������������������������������������������������������ͼ

MGR Codes: 1=AWS3880  2=AWS3274  3=AWS2821  4=AWSTFA   5=AWSPCSRV 6=AWS5080

7=AWS3215  8=AWS3088  9=LAN3274  A=AWS2703  B=AWSTAPE  C=AWS9346  D=CHAN370

���Ŀ

[XXX] �602� is the S/390 address

�����   �Ŀ

[#]           �8� is the AWS3088 manager code from the MGR Codes list

���   ������������Ŀ

[AAAAAAAA.AAAAAA #] �RALVMPA.NETe� is the remote connection definition

��������������

PARAMETER SYNTAX

�����������������Ŀ

�NNNNNNNN.SSSSSS #� REMOTE CONNECTION DEFINITION

�������������������

��������Ŀ

�NNNNNNNN� is eight alphanumeric character NODE name

����������

�.� is required delimiter if SUBCHANNEL EXTENSION is specified

�������Ŀ

�SSSSSS� is six alphanumeric character SUBCHANNEL EXTENSION

���������Ŀ

�#� is LANA (Local Area Network Adapter) number (0|1)

���

��������Ŀ

�NNNNNNNN� NODE NAME may be a CTC NODE NAME

����������

the name of a remote P/390

in which case the S/390 address is a CTC device

and a SUBCHANNEL EXTENSION must be specified

ordinarily the CTC NODE NAME is the [Local Area NetID]

entry on the UPDATE SYSTEM ENVIRONMENT [PF4] configuration

option on the remote P/390 system

������Ŀ

�SSSSSS� SUBCHANNEL EXTENSION (TO CTC NODE NAME)

��������

the SUBCHANNEL EXTENSION is the mechanism used to make

the CTC connection (a 3088 subchannel) and the same

SUBCHANNEL EXTENSION must appear in the configurations

of both P/390 systems

�Ŀ

�#� LANA NUMBER (OPTIONAL)

���

the LANA NUMBER is used to select a LANA in the case

where there is more than one LANA installed

use [0] to select the PRIMARY adapter

use [1] to select the SECONDARY adapter

�� P/390 RALVMPA Ŀ

� 602 RALVMPC.PVMe]�������ķ            ������������������������ LEGEND Ŀ

� 603 RALVMPC.PVMo]�������Ķ            �  PVM = half duplex PVM         �

� 604 RALVMPB.PVM ]�����Ŀ �            �  PVMe|PVMo = full duplex PVM   �

� 680 RALVMPB.NETe]���ķ � �            �                                �

� 681 RALVMPB.NETo]���Ķ � �            �  NET = half duplex RSCS        �

� 6A0 RALVMPB.TSAF]�Ŀ � � �            �  NETe|NETo = full duplex RSCS  �

�������������������  � � � �            �                                �

� � � �            �  TSAF = half duplex TSAF       �

�� P/390 RALVMPB Ŀ  � � � �            ����������������������������������

� 6A0 RALVMPA.TSAF]��� � � �

� 680 RALVMPA.NETe]���Ķ � �     �� P/390 RALVMPC Ŀ

� 681 RALVMPA.NETo]���Ľ � ������[600 RALVMPA.PVMe �

� 600 RALVMPA.PVM ]������� ������[601 RALVMPA.PVMo �

� 601 RALVMPC.PVM ]��������������[602 RALVMPB.PVM  �

� 682 RALVMPC.NET ]��������������[680 RALVMPB.NET  �

�������������������              �������������������

AWS3088 INVOCATION PARAMETERS

awsstart AWS3088.EXE n

�� NBIOS ( #sessions , #commands , #names ) Ŀ

�                                            �

�  0BIOS ( #sessions , #commands , #names )  �

�� 1BIOS ( #sessions , #commands , #names ) ��

NETID ( xxxxxxxx )

CTCBUF ( # )

PVMBUF ( # )

DEBUG ( 0|1|2 )

�� NBIOS ( #sessions , #commands , #names ) Ŀ    NETBIOS RESET PARAMETERS

�  0BIOS ( #sessions , #commands , #names )  �

�� 1BIOS ( #sessions , #commands , #names ) ��

use NBIOS or 0BIOS to override default NETBIOS NCB.RESET parameters for

LANA[0] (PRIMARY adapter)

use 1BIOS to override default NETBIOS NCB.RESET parameters for LANA[1]

(SECONDARY adapter)

unless overridden the default values are calculated:

#sessions = 1 + (number of unique remote NODE NAMES)

#commands = 2 + 2*(number of AWS3088 devices) + (CTCBUF+PVMBUF)

#names = (number of LANA used for PVM) + (number of LANA used for CTC)

WARNING: AWS3088 MAY NOT OPERATE RELIABLY IF NOT ENOUGH RESOURCES ARE

ALLOCATED TO ACCOMMODATE THE MAXIMUM NUMBER OF ACTIVE DEVICES

CTCBUF ( # )  NUMBER OF 4K DATA BUFFERS TO USE WITH EACH CTC CONNECTION

PVMBUF ( # )  NUMBER OF 4K DATA BUFFERS TO USE WITH EACH PVM CONNECTION

both CTCBUF and PVMBUF are allocated within a single 64K segment so the

total (CTCBUF+PVMBUF) must be 15 or less

default values depend upon the number of LANA used for CTC and/or PVM

(total ranges from 12 to 15)

if no PVM devices are configured, then all 15 buffers will be used by

CTC devices (likewise if no CTC devices then PVM devices get everything)

NETID ( xxxxxxxx ) LOCAL NODE NAME

used to override the default [Local Area NetID] found in the UPDATE

SYSTEM ENVIRONMENT [PF4] section of the configurator

the actual names used to establish NETBIOS sessions are formed by

-  appending a floating "-3088" suffix for CTC devices

-  appending a right justified "-PVMPC" suffix for PVM devices