IBM 3101 Model 2X

Introduction

The IBM 3101 Display Terminal Models 20, 22, or 23 can be emulated

using Softerm by selecting IBM 3101-2X for the terminal emulation

when generating a configuration in terminal setup.  Softerm

supports all the general capabilities of the IBM 3101 Model 2X

including Character or Block Mode Transmission, Full Duplex

Operation, Half Duplex Operation with full duplex communications,

Auxiliary Printer Interface, Buffer Print Operations, full 128

character set and 80 X 24 display, Program Function Keys, Model 2X

unique function keys, Keyboard lock/unlock, Cursor control, Erase

functions, Tab operations, Edit Functions, Auto New Line, Scroll

On/Off, CR/CR-LF, Auto Line Feed, and Audible Alarm.

Features

The display format of the IBM 3101 Model 2X display terminal is 24

lines by 80 characters with the ability to display all 128 ASCII

character including 32 control codes in the Transparent Mode.  The

3101 also includes an operator information area at the bottom of

the display screen as an operation interface.  It displays various

modes and status of the 3101 communications and operations.

Softerm includes a special status line which provides similar

information concerning the operating modes of the 3101 emulation.

The IBM 3101 Model 2X functions in either the character or block

mode selectable by a Softerm Terminal Setup configuration option.

In the character mode, a character is immediately transmitted when

a keyboard key is pressed.  In the block mode, a data stream is

transmitted, which has been edited and stored when the SEND key is

pressed.

In character mode, Softerm supports the full duplex and half

duplex operation mode when using a full duplex communications

facility.  In block mode, Softerm supports the half duplex

operation mode with a full duplex communication facility.  Half

duplex operation using a half duplex communication facility with

or without reverse channel is not supported.  Thus operation of

the 3101 emulation with a 202-type modem with switched RTS

(request to send) is not supported.

The Softerm terminal emulation options Auto Wrap and Page Mode

correspond to IBM 3101 setup switch options Automatic New Line and

Scroll On/Off respectively.

Setup switch options available on the IBM 3101 Model 2X and not

supported by Softerm include Mono character set selection,

Interface Selection, Modem Control, Reverse Channel Control, and

Reverse Video On/Off.

Softerm supports all the 3101 Model 2X keyboard functions.  The

AUX key capability is provided by the Softerm Ctrl PrtSc Printer

Enable/Disable Function.

Host initiated functions which Softerm recognizes but ignores

include Logical Keyboard Lock and Cancel.

IBM 3101-2X                  - 1 -             Terminal Emulation

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������� IBM 3101-2X Emulation Options ����Ŀ

Line Turnaround Character: CR ($0D)

Operating Mode: Character

Null Suppress: Yes

Send Key = Send Line: No

Send Key Definition: Alt Enter

Function Keys (PF9-PF24): No

Enable Print Pass Thru: No

Enable Block Transmit Delay: No

Block Delay (1/10 Secs.): 10

Block Size (Bytes): 256

Enable Autotab: No

The emulation option Line Turnaround Character selects the line

turnaround and block terminator character.  The character selected

can be CR ($0D), EOT ($04), ETX ($03), or XOFF ($13) depending on

the requirements of the host computer.  If the XOFF character is

selected, the user should insure that the Terminal Emulation

Parameter Receive Pacing is set to None.

The emulation option Operating Mode selects the initial operating

mode for the terminal when online operation is initiated.  Block

or Character mode may be selected.  If Block mode is selected, the

characters entered from the keyboard are stored in a buffer

corrected and edited by the operator, and then transmitted to the

host when the SEND key is pressed.  The AUTO WRAP option is forced

if block mode is selected.  Softerm can also be toggled between

the block and character modes during online operation using the

3101 Status Display initiated with the Alt T keyboard function.

The emulation option Null Suppress allows trailing nulls to be

suppressed when a block mode transmission is performed.  This

option is valid only for block mode operation and is selected by

Yes.  If null suppression is not required, No should be selected.

The emulation option Send Key = Send Line is used to activate the

SEND LINE key function when the SEND key is pressed.  If Yes is

selected, the SEND key will function as the SEND LINE key.  If No

is selected, the SEND key will function as the SEND PAGE key.

The emulation option Send Key Definition allows you to define the

key which will initiate a send operation.  If Enter is selected,

then pressing the Enter key will initiate a send operation and

pressing Alt Enter will generate a CR, CR-LF, or LF sequence.  If

Alt Enter is selected, then pressing the Enter key will generate a

CR, CR/LF, or LF sequence, and pressing the Alt Enter key will

initiate a send operation.

The remaining options are normally used when the IBM 3101 Model 2X

emulation is used with Tymnet's 3270 protocol conversion

capability.  These options should be set to Yes when using this

service.  The option Function Keys (PF9-PF24) provide a Tymnet

compatible extension to support the Program Function Keys PF9-PF24

normally available on a 3270 terminal.  The option Enable Print

IBM 3101-2X                  - 2 -             Terminal Emulation

Pass Thru provides the capability for the host to redirect the

terminal data stream to the printer instead of the display through

a host initiated sequence.  The three character sequence (ESC # 6)

when received from the host activates print pass thru if it is

enabled, and the sequence (ESC # 1) when received from the host

deactivates print pass thru.  The option Enable Block Transmit

Delay is provided so large block transmissions can be divided into

smaller blocks with a time delay after each block to compensate

for satellite network delay time in international use.  The

options is used in conjunction with the options Block Delay (1/10

Secs.)  and Block Size (Bytes).  The Block Size defines the

maximum number of characters which will be transmitted before

initiating a delay, and the Block Delay defines the amount of

delay in one tenth second increments.

Operational Characteristics

During block mode operation, when the display screen is currently

unformatted (no field attributes are present), and a start field

command is received, data will be displayed as it is received.

However, all screen positions for certain types of fields are not

immediately updated unless characters are received for each

position.  After no additional data has been received for a

timeout period of approximately 1 second, the entire screen will

be checked and any additional updating required will be performed.

If the keyboard becomes inadvertently locked through a received

keyboard lock command from the host without a subsequent keyboard

unlock command, the Alt R keyboard function may be used to unlock

the keyboard without causing a disconnect.  However, this function

will also rewrite the screen and can be used whenever the current

screen display appears to be incorrect due to an error condition.

Operator Status Line Display

The IBM 3101-2X terminal emulation includes a status display

similar to the Operator Information Area displayed on line 25 of

the 3101.  The status information displayed includes the current

Row and Column, Block or Char Mode, and Program or Insert sub-

modes.  The status line can be displayed by pressing Alt T during

online terminal operation.

While the status is displayed, the Operating Mode of the 3101

emulation can be toggled by pressing the B key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking Right Arrow appears in the first column of the

status line.  For example, to toggle from the half duplex

character mode to block mode, first display the status by entering

Alt T and then press the B key to change the Char Mode to Block

Mode.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

IBM 3101-2X                  - 3 -             Terminal Emulation

IBM 3101-2X Keyboard Functions

The following table defines the standard keyboard functions for

the IBM 3101 Model 2X terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt Enter		Send

Shift Enter		Send Line

Ctrl Enter		Send Page

Alt Shift Enter	Send Message

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt PrtSc		PRINT LINE

Shift PrtSc		PRINT PAGE

Ctrl PrtSc		AUX

Alt Shift PrtSc	PRINT MESSAGE

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Backspace		BS or DEL

Shift Backspace	DEL or BS

Alt A			ATTR

Alt B			Break

Alt P			PRGM MODE

Alt R			Reset Keyboard Lock, Rewrite Screen

Alt T			Display 3101 Status Line

Tab			Tab

Shift Tab		Back Tab

Home			Home

PgUp			Erase Input

Ctrl PgUp		Erase All

PgDn			Erase EOS

End			Erase EOL

Del			DEL CHAR

Alt Del		DEL LINE

Ins			INS CHAR

Alt Ins		INS LINE

F1	PF1		F5	PF5

F2	PF2		F6	PF6

F3	PF3		F7	PF7

F4	PF4		F8	PF8

IBM 3101-2X                  - 4 -             Terminal Emulation

The following function keys are available if the Function Keys

(PF9-PF24) option is set to Yes.

F9		PF9		Shift F7	PF17

F10		PF10		Shift F8	PF18

Shift F1	PF11		Shift F9	PF19

Shift F2	PF12		Shift F10	PF20

Shift F3	PF13		Alt F1		PF21

Shift F4	PF14		Alt F2		PF22

Shift F5	PF15		Alt F3		PF23

Shift F6	PF16		Alt F4		PF24

IBM 3101-2X                  - 5 -             Terminal Emulation