Hazeltine 1420 Video Display Terminal

Introduction

The Hazeltine 1420 Video Display Terminal can be emulated using

Softerm by selecting HAZ1420 for the terminal emulation when

generating a configuration in terminal setup.  The Hazeltine 1420

general capabilities supported by Softerm include the full 128-

character ASCII set with 80X24 display; asynchronous character

transmission mode; visual attributes; full keyboard emulation; and

auxiliary port printer support.

Features

Softerm recognizes and processes all Hazeltine operational

commands.

After the last standard screen in the Terminal Parameters mode in

Terminal Setup, the following screen will be displayed:

������ Hazeltine 1420 Emulation Options �����Ŀ

Automatic Line Feed: Yes

Font: Upper and Lower Case

Lead-In: ESC

Wraparound: Yes

The emulation option Automatic Line Feed may be toggled to Yes or

No.  If set to Yes, all received carriage returns automatically

cause the cursor to move to the first character position of the

next line.  If the cursor was on the last displahable character

row, carriage returns will scroll the display.  If set to No, all

received carriage returns move the cursor to the first character

position of the same line.  Received line feed characters will

move the cursor down one line.  If the cursor was on the last

displayable character row, line feed characters cause the display

to scroll.

The emulation option Font may be toggled to Upper and Lower Case

or Upper Case.  If set to Upper and Lower Case, the full 128-

character ASCII set is available for transmission and the 95-

character alpha/numeric set is available for display.  The Caps

Lock key may be used for all upper-case operation.  If set to

Upper Case, all lower-case alpha characters generated from the

keyboard are converted to upper-case for transmission and display

and all received characters are converted to upper-case for

display.  The Caps Lock key is disabled.

The emulation option Lead-In may be toggled to ESC or Tilde to

select the remote command lead in character.

The emulation option Wraparound may be toggled to Yes or No.  If

set to Yes, the cursor moves to the first column of the next row

without requiring a carriage return or line feed, whenever a

character is entered in the last column of a row, either from the

keyboard or as received data.  If the cursor was on the last row,

the display scrolls up.  If set to No, the cursor will remain at

Hazeltine 1420               - 1 -             Terminal Emulation

the last column position of the row.  If data continues to be

entered or received, the characters overwrite the last displayed

character on the row.

Hazeltine 1420 Keyboard Functions

The following table defines the standard keyboard functions for

the Hazeltine 1420 terminal emulation:

Keyboard		Function

Enter			CR

Alt Enter		Line Feed

Home			Home

Backspace		BS

Tab			Next Foreground Field

Shift Tab		Horizontal Tab

Alt PgUp		Clear Screen

Alt PgDn		Clear Foreground

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt F10		Function 0

F1			Function 1

F2			Function 2

F3			Function 3

F4			Function 4

F5			Function 5

F6			Function 6

F7			Function 7

F8			Function 8

F9			Function 9

F10			Function 10

Alt F1			Function 11

Alt B     Break

Hazeltine 1420               - 2 -             Terminal Emulation