TeleVideo 921

Introduction

The TeleVideo Model 921 terminal can be emulated using Softerm by

selecting TeleVideo 921 for the terminal emulation when generating

a configuration in terminal setup.  Softerm supports all the

general capabilities of the TeleVideo 921 including the full 128

character set and 80 X 24 display, embedded and non-embedded

visual attributes, Tab and Back Tab, editing features, cursor

controls, programmable function keys, conversational full or half

duplex transmission, block mode transmission, and auxiliary port

control including page print, buffered transparent, and buffered

with screen copy.

Features

The display format of the TeleVideo 921 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 15 special graphics characters and 32 control

codes in the Monitor Mode.  Both a United States and United

Kingdom character sets are supported.  The TeleVideo 921 also

displays a 25th status line which is simulated by Softerm.

Softerm supports all available keyboard functions.

Functional commands recognized and processed but ignored by

Softerm include Back Page, Advance Page, Keyclick On, Keyclick

Off, Turn Screen Off, Turn Screen On, Auto Page On, and Auto Page

Off.

Softerm includes support for the Printer Port if the terminal

configuration includes a printer.  If either an Enable

Bidirectional Port, Extension Mode On, or Transparent Print On

remote command is received, all subsequent data is sent to the

printer until a Disable Bidirectional Port, Extension Mode Off, or

Transparent Print Off command is received.  In the transparent

mode data is sent only to the printer and not displayed, otherwise

the received data is both displayed and printed.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� TeleVideo 921 Emulation Options ����Ŀ

Operating Mode: Character

Edit Mode: Duplex

Page Attributes: No

Embedded Attributes: Yes

Cursor Down Key Sends: Ctrl V

Character Set: US ASCII

Margin Bell: No

Send Key = Send Line: No

TeleVideo 921                - 1 -             Terminal Emulation

The emulation option Operating Mode selects the transmission mode

of the terminal and may be selected as either Character or Block.

In Block mode, characters entered from the keyboard are stored in

a buffer which may be corrected and edited by the operator, and

then transmitted to the host when the SEND key is pressed.

The emulation option Edit Mode sets the edit keys for local

operation only if Local is selected and the Duplex Edit Mode if

Duplex is entered.  In Duplex Edit Mode the edit keys will be

transmitted as alphanumeric keys and operated on according to the

current Full or Half Duplex mode.  The editing keys are TAB, BACK

TAB, CHAR INSERT, CHAR DELETE, LINE INSERT, LINE DELETE, LINE

ERASE, PAGE ERASE, BACKSPACE, HOME, CURSOR UP-DOWN-RIGHT-LEFT,

CLEAR SPACE, CE, SEND, and PRINT.

The emulation option Page Attributes if set to Yes will allow the

Set Visual Attribute command to set the visual attributes from the

current cursor position to the end of the screen.  If this option

is set to No, the Set Visual Attribute command affects only the

current line.

The emulation option Embedded Attributes if set to Yes will cause

visual attributes to occupy a screen position.  If this option is

set to No, visual attributes will not occupy a screen position and

the screen position can be used to display data.

The emulation option Cursor Down Key Sends allows the character

code transmitted when the cursor down key is pressed to be

selected.  If Ctrl V is selected, a cursor down code ($16) is

transmitted when the cursor down key is pressed.  If Ctrl J is

selected, a line feed code ($0A) is transmitted.

The emulation option Character Set allows the character set used

to be selected.  It may be selected as either US ASCII for the

American version or as UK English for the English version.

The emulation option Margin Bell if set to Yes will cause the bell

to sound whenever data is entered at the margin bell column.  The

margin bell column defaults to column 72.  If this option is set

to No, the margin bell is not used.

The emulation option Send Key = Send Line is used to activate the

SEND LINE key function when the SEND key is pressed.  If Yes is

selected, the SEND key will function as the SEND LINE key.  If No

is entered, the SEND key will function as the SEND PAGE key.

TeleVideo 921 Function Key Definition

When editing of the TeleVideo 921 emulation options is complete,

pressing the Alt Enter key will cause the following screen to be

displayed:

TeleVideo 921                - 2 -             Terminal Emulation

��� TeleVideo 921 Function Key Definitions �Ŀ

Unshifted              Shifted

Mode Definition       Mode Definition

F1    1  �cA�@�cM�          1  �cA�`�cM�

F2    1  �cA�A�cM�          1  �cA�a�cM�

F3    1  �cA�B�cM�          1  �cA�b�cM�

F4    1  �cA�C�cM�          1  �cA�c�cM�

F5    1  �cA�D�cM�          1  �cA�d�cM�

F6    1  �cA�E�cM�          1  �cA�e�cM�

F7    1  �cA�F�cM�          1  �cA�f�cM�

F8    1  �cA�G�cM�          1  �cA�g�cM�

F9    1  �cA�H�cM�          1  �cA�h�cM�

F10   1  �cA�I�cM�          1  �cA�i�cM�

F11   1  �cA�J�cM�          1  �cA�j�cM�

F12   1  �cA�K�cM�          1  �cA�k�cM�

F13   1  �cA�L�cM�          1  �cA�l�cM�

F14   1  �cA�M�cM�          1  �cA�m�cM�

F15   1  �cA�N�cM�          1  �cA�n�cM�

F16   1  �cA�O�cM�          1  �cA�o�cM�

The fields displayed for each function key definition include the

Mode and Definition which define a communication mode and a

message of up to 8 characters to be transmitted by that function.

The Mode field defines a communication mode for the function key

according to the following values:

Mode	Effect

1	Send message to computer

2	Send message to terminal

3	Send message to both computer and terminal

The Definition is the message to be transmitted by that function

key.  The message can contain any combination of up to 8

alphanumeric characters, control codes, or escape sequences.

Operator Status Line Display

The TeleVideo 921 terminal emulation includes a status line which

may be displayed on the 25th line by pressing the Alt T key.  The

status information displayed includes Row and Column, Local or

Duplex edit mode, Character or Block mode, H.I.  (Half Intensity)

mode, Protect mode, Lock (Keyboard Lock), and Mont (Monitor Mode).

The Block or Character mode setting can be toggled while the

status is displayed by pressing the B key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking RIGHT ARROW appears in the first column

position.  For example, to toggle from the Block mode to the

Character mode, first display the status by entering Alt T and

then press the B key.  Pressing the E key will cause the Local or

Duplex editing mode to be toggled.  Pressing the H key will cause

the H.I.  mode to be toggled between On and Off.  Pressing the M

key will cause the Mont (Monitor Mode) to be toggled between On

and Off.  Pressing the P key will cause the PROTECT mode setting

to be toggled between On and Off.  Pressing the K key will cause

the LOCK (Keyboard Lock) setting to be toggled between On and Off.

TeleVideo 921                - 3 -             Terminal Emulation

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

TeleVideo 921 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 921 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt Enter		SEND

Shift Enter		SEND LINE ALL

Ctrl Enter		SEND PAGE ALL

Del			CHAR DELETE

Alt Del		LINE DELETE

Ins			CHAR INSERT

Alt Ins		LINE INSERT

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Ctrl Esc		LOC ESC

Shift PrtSc		PRINT

Alt Shift PrtSc	SHIFT PRINT

Backspace		BACKSPACE or DEL

Shift Backspace	DEL or BACKSPACE

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Tab			TAB

Shift Tab		BACK TAB

Alt A			Send Answerback Message

Alt B			BREAK

Alt F			FUNCT

Alt T			Display 921 Status Line

Home			HOME

PgUp			CLEAR SPACE

Ctrl PgUp		CLEAR ALL (Nulls)

Alt PgUp		PAGE

Alt Ctrl PgUp		SHIFT PAGE

End			LINE ERASE (Spaces)

Ctrl End		SHIFT LINE ERASE (Nulls)

Alt End		CE

Alt Ctrl End		SHIFT CE

PgDn			PAGE ERASE (Spaces)

Ctrl PgDn		SHIFT PAGE ERASE (Nulls)

F1		F1	Shift F1	SHIFT F1

F2		F2	Shift F2	SHIFT F2

F3		F3	Shift F3	SHIFT F3

F4		F4	Shift F4	SHIFT F4

F5		F5	Shift F5	SHIFT F5

TeleVideo 921                - 4 -             Terminal Emulation

F6		F6	Shift F6	SHIFT F6

F7		F7	Shift F7	SHIFT F7

F8		F8	Shift F8	SHIFT F8

F9		F9	Shift F9	SHIFT F9

F10		F10	Shift F10	SHIFT F10

Alt F1		F11	Alt Shift F1	SHIFT F11

Alt F2		F12	Alt Shift F2	SHIFT F12

Alt F3		F13	Alt Shift F3	SHIFT F13

Alt F4		F14	Alt Shift F4	SHIFT F14

Alt F5		F15	Alt Shift F5	SHIFT F15

Alt F6		F16	Alt Shift F6	SHIFT F16

TeleVideo 921                - 5 -             Terminal Emulation