Kimtron KT-7 Display Terminal

Description

The Kimtron KT-7 display terminal can be emulated using Softerm by

selecting Kimtron KT-7 for the terminal emulation when generating

a configuration from the Setup Options menu.  The Kimtron KT-7

general capabilities supported by Softerm include the full 128-

character ASCII set; international and line drawing character

sets; 2 or 4 pages total display memory (2 pages = 1 page of 48

lines or 2 pages of 24 lines; 4 pages = 1 page of 96 lines, 2

pages of 48 lines or 4 pages of 24 lines); 20 function keys (10

unshifted, 10 shifted); transparent and bidirectional print modes;

addressable user line; and hidden/embedded attributes.

Features

The default display format of the Kimtron KT-7 is 2 pages of 24

lines each.  The user or remote system may select either a normal

screen (white on black) or a reverse screen (black on white).

Operating mode is also host/user selectable as block or

conversational (full or half duplex) mode.  Host/user addressable

User Line, Answerback Message and Function keys.  The user also

may place the terminal in native mode (Kimtron KT-7) or in

Televideo 920E mode.

Softerm supports all commands with the following exceptions, which

it recognizes but does not support: Keyclick On/Off, Load Time,

Blank Screen and Normal Screen.

Kimtron KT-7 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������ Kimtron KT-7 Emulation Options ����Ŀ

Emulation: Kimtron KT-7

Attributes: Hidden

Line/Page Attributes: Line

Edit Mode: Duplex Edit

Auto New Line: Yes

Maximum Display Memory: 2

Line 25: Status

The emulation option Emulation may be toggled to Kimtron KT-7 or

Televideo 920E.  If set as a Kimtron KT-7, the terminal operates

as described above.  If set to Televideo 920E, the terminal

operates as a Televideo 920E terminal, and responds correctly to

920E commands.

The emulation option Attributes may be toggled to Hidden or

Embedded.  If set to Embedded, visual attributes occupy a space on

the screen.  If set to Hidden, visual attributes are not embedded

and do not occupy a space.

Kimtron KT-7                 - 1 -             Terminal Emulation

The emulation option Line/Page Attributes may be toggled to Line

or Page.  If set to Line, video attributes are enabled for the

line on which the cursor is located.  If Page is selected, video

attributes are enabled from the cursor to the next attribute or

end of page which ever occurs first.

The emulation option Edit Mode may be toggled to Duplex Edit or

Local Edit.  If Duplex Edit is selected, the cursor control keys

will operate in terminal communication mode like any key.  If set

for Local, all cursor control keys will operate in Local mode

only, regardless of the terminal communication mode.

The emulation option Auto New Line may be toggled to Yes or No.

If set to Yes, the terminal will perform a CR and LF operation

whenever data is entered on screen column 80.  If set to No, the

cursor will stay on column 80 until a cursor control code is

received (i.e up,down,CR,LF, etc).  Data is overwritten at column

80.

The emulation option Maximum Display Memory may be toggled to 2 or

4.  If 2 is selected, the terminal uses a maximum of 48 lines.

These 48 lines can be used as either 1 page of 48 lines or 2 pages

of 24 lines each.  If 4 is selected, then the terminal uses a

maximum of 96 lines.  These 96 lines can be configured as 1 page

of 96 lines, 2 pages of 48 lines each, or 4 pages of 24 lines

each.

As an enhancement to the Kimtron KT-7, Softerm supports 2

additional escape code sequences: Esc D and Esc H.

Esc D allows the host to write to a page without displaying it.

For instance, if the current active page is page 1 and the host

wishes to write to page 2, entering Esc D would keep the current

page displayed, while all received data was processed on page 2

without being displayed.  The status line and user line, however,

are still updated in the usual manner.

Esc H transfers control back to current displayed page.

Note: For this mode to work, 2 pages of 24 lines or 48 lines must

be configured.  If 1 page or 4 pages is configured, the command is

ignored.

The emulation option Line 25 may be toggled to Status, User Line

or None, and dtermines what will be displayed on line 25 of your

screen.  If set to Status, the 25th line will display the KT-7

status line; if set to User Line, the 25th line will display the

KT-7 User Line; and if set to None, nothing will be displayed on

line 25.

KT-7 Operator Status Line Display

The Kimtron KT-7 terminal emulation includes a status line which

may be displayed on the 25th line.  Displayable parameters include

the time, prefaced with either "A" or "P" to show AM or PM; Block

or Character (Conversation) mode; Keyboard Lock or Ready; Monitor

mode; Local or On Line; number of display pages selected (2 or 4),

current page number, current row, and current column.

Kimtron KT-7                 - 2 -             Terminal Emulation

While this menu is displayed, pressing B will toggle between Block

and Character modes.  The status line will show either BLK or the

current duplex mode (FDX or HDX).  Pressing K will toggle Keyboard

Lock on or off, and the status line will show either KB LOCK or

READY.  Pressing D will toggle the Edit Mode (described in the

Emulation Options section), however, the setting is not reflected

on the status line.  Pressing W will toggle Write Protect mode;

however, although the setting is not reflected on the status line,

characters typed at the Online screen may be displayed with a

different attribute.  Pressing M will toggle Monitor mode.  If

Monitor mode is On, the status line will display MONIT.  Pressing

P will toggle between Variable and Protect modes, and the status

line will display either VAR or PRO.  With Protect mode on, the

cursor will tab past any data entered with Write Protect mode set.

Pressing U will display the User Line in place of the status line

and the menu will be cleared from the screen.  Pressing C will

toggle the Margin Bell on and off.

To clear the menu, press either Alt T or Alt Esc.  Pressing Alt T

will clear the menu and leave the status line displayed.  Pressing

Alt Esc will clear the menu and the status line.

Kimtron KT-7 Keyboard Functions

The following table defines the standard keyboard functions for

the Kimtron KT-7 terminal emulation:

Keyboard		Function

Enter			Return

Ctrl Enter		Newline

Alt Enter		XMIT Page

Alt Shift Enter	XMIT Unprotected

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Home			Cursor Home

BackSpc		Backspace

Tab			Column/Field Tab

Shift Tab		BackTab

Ctrl Tab		Clear All Tabs

Alt Tab		Set Tab

Alt Shift Tab		Clear Tab

Alt F1			Print Time of Day

Ins			Insert Character

Alt Ins		Insert Line

Del			Delete Character

Alt Del		Delete Line

Shift PrtSc		Page Print All

Ctrl PrtSc		Page Print Unprotected

Kimtron KT-7                 - 3 -             Terminal Emulation

Use Shift + key for shifted functions:

F1 - F10		Function Keys 1 - 10

Use Ctrl + key for these functions:

F1		Block Mode

F2		Conversation Mode

F3		Half Duplex Mode

F4		Full Duplex

F5		Protect Mode On

F6		Protect Mode Off

F7		Write Protect On

F8		Write Protect Off

F9		Line Draw On

F10		Line Draw Off

Kimtron KT-7                 - 4 -             Terminal Emulation