AT&T 610 Business Communications Terminal

Description

The AT&T 610 Business Communications Terminal can be emulated

using Softerm by selecting ATT610 for the terminal emulation when

generating a configuration in terminal setup.  The AT&T 610

general capabilities supported by Softerm include the full 128

character ASCII set, with support for double high/wide characters;

United Kingdom character set, and a special character and line

drawing character set; simulated 80 or 132 character host writable

status line; visual attributes; full keyboard emulation including

cursor control, editing, word processing, device control, and

function keys; 98 or 103 key keyboard; auxilliary port printer

support; and support for 132 columns of video display.

Features

The display format of the AT&T 610 is 27 lines by 80 or 132

characters with the capability to display 232 characters including

the 128 ASCII character set, 1 additional United Kingdom

character, 31 special characters and line drawing characters, 7

Securities Industry characters, and 65 Mosaic characters.

Softerm supports display of the standard ASCII character set, the

special characters and line graphic characters, 2 additional

Securities Characters, and 5 of the Mosaic characters.  The AT&T

610 also displays a 25th line called the Status Line, which is

used to as a message line separate from the data space, providing

a one line window for dialog between the operator and the host.

Softerm provides full support for the status line capabilities.

Lines 26 and 27 of the AT&T 610 are used for screen labels.  These

two lines are not supported by Softerm but are available by

pressing Alt-F on the keyboard.  The screen labels will appear on

lines 23 and 24 of the Softerm display.  A Ctrl-Fkey combination

will change the screen labels to the root labels listed.  Root

labels supported by Softerm are print labels, user function

labels, and system function labels.  Root labels not supported by

Softerm are I/O labels, setup labels, and graphics labels.

Softerm supports all setup options available on the AT&T 610

except I/O Card, Cartridge, Volume, and Key Click.

Softerm recognizes and processes all host-initiated AT&T 610

operational commands with certain exceptions:

Host-initiated AT&T 610 operational commands which Softerm

recognizes but ignores include Set Cursor to Blink, Set Cursor to

Steady, Set Cartridge to "Used", Set Cartridge to "Idle", Turn On

Key Click, Turn Off Key Click.

AT0T 610                     - 1 -             Terminal Emulation

Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

��������� AT&T 610 Emulation Options �������Ŀ

Check Parity: No

Local Echo: OFF

Monitor Mode: OFF

Alternate Enter Key = CR: Yes

Alternate Enter Key Sends:

Columns: 80

Keyboard Size: 103 Key

Scroll: Jump

Reverse Video: No

User Function Key Labels: OFF

Local Horizontal Scroll Increment: 1

Horizontal Scroll Delay (1/10 Secs.): 2

Clear Options To Default Values: No

( Includes Line Parameters )

The emulation option Check Parity determines if the received

character will be checked for parity errors.  If this option is

set to Yes and a parity error occurs, the SUB character will be

displayed in place of the received character.

The emulation option Local Echo will set the duplex.  If set to

Off, characters will be sent on line only.  If set to On,

characters will be sent on line and to the display.

The emulation option Monitor Mode determines how escape sequences

and control characters will be acted upon when received.  If set

to Off, all escape sequences and control characters will be acted

upon normally.  If set to On, all characters including control

characters will be displayed.  The control characters LF, VT, and

FF will be displayed and acted on.  When Monitor Mode is set to

On, the autowrap feature is temporarily enabled when the cursor

reaches the end of the line.

The emulation option Alternate Enter Key = CR will cause a CR to

be transmitted when a shift enter is pressed at the keyboard, if

the option is set to Yes.  If the option is set to No, the buffer,

which is entered in the following option, will be sent.

The emulation option Alternate Enter Key Sends allows the operator

to enter a four-character string to be sent when a Shift-Enter is

pressed at the keyboard.  The previous emulation option must be

set to No to allow the alternate enter key buffer to be sent.

The emulation option Columns determines how many columns are

available in the data space.  If set to 132, the screen will

scroll left when the right margin is reached until column 132.  A

carriage return will cause the screen to scroll right, returning

the cursor to column one.  Softerm provides local keyboard

functions to allow for scroll left/right and home left/right.

AT0T 610                     - 2 -             Terminal Emulation

The emulation option Keyboard Size is set for the appropriate AT&T

keyboard.  Only keys available on that keyboard will be allowed

for transmission.

The emulation option Scroll determines how the screen will scroll

when the bottom margin is reached.  If set to Jump, the screen

will scroll immediately.  If set to 6 Lines/Second, the scroll

rate will be slowed.

The emulation option Reverse Video will display light characters

on a dark background if set to No.  If set to Yes, the screen will

appear with dark characters on a light background.

The emulation option User Function Key Labels, if set to On, will

cause the host or user-supplied User Function Key Labels to appear

in place of the Default Function Key Labels when the Alt-F key

combination is pressed.  The root functions may still be obtained

by using a Ctrl-Fkey combination.

The emulation option Local Horizontal Scroll Increment determines

how many columns the screen will scroll horizontally when the

Softerm local scroll and home functions are used in 132-column

mode.

The emulation option Horizontal Scroll Delay is the delay time in

tenths of a second that the horizontal scroll will delay when the

cursor reaches the right or left margin in 132-column mode.

The emulation option Clear Options To Default Values will clear

all options, including the line parameters, to default values if

the Alt-Enter key combination is pressed with this option set to

Yes.  The default values are:

Check Parity:  No			       Columns:  80

Local Echo  :  Off		 Keyboard Size:  103 Key

Monitor Mode:  Off			Scroll:  Jump

Alternate Enter Key = CR : Yes     Reverse Video:  No

Alternate Enter Key Sends: blank

User Function Key Labels : Off

Local Horizontal Scroll Increment   : 1

Horizontal Scroll Delay (1/10 Secs.): 2

Parity:  Even

Speed :  1200

AT0T 610                     - 3 -             Terminal Emulation

After the preceding options have been specified, press the Alt

Enter key and the following additional screen is displayed:

����� AT&T 610 Function Key Definitions ���Ŀ

Key        Label           String

F1:           F1           �ESC�Oc

F2:           F2           �ESC�Od

F3:           F3           �ESC�Oe

F4:           F4           �ESC�Of

F5:           F5           �ESC�Og

F6:           F6           �ESC�Oh

F7:           F7           �ESC�Oi

F8:           F8           �ESC�Oj

Clear Functions To Default Values: No

This menu allows the operator to re-define User Function Keys.

The Label can consist of two strings, each up to eight characters

long, that will be displayed when the User Function Key Labels are

displayed through the Alt F or Ctrl Function-key combinations.

The String consists of up to thirty-five characters that will be

transmitted when the matching function key is pressed.  In the

following example, function key F1 is remapped to set Monitor Mode

to On:

Key  Label         String

F1:  Monitor On	<<Esc>>[13h

The option to reset function keys to default values will restore

original values to function key buffers.

AT&T 610 Keyboard Functions

The following table defines the standard keyboard functions for

the AT&T 610 terminal emulation:

Keyboard	Function

Enter		CR, LF or CR/LF

Shift Enter	Alternate Enter

Backspace	Backspace

Home		Home

Tab		Tab

Shift Tab	Back Tab

Right Arrow	Cursor Forward

Left Arrow	Cursor Backward

Up Arrow	Cursor Up

Down Arrow	Cursor Down

Ctrl Break	Soft Reset

F1-F8		Default Functions

Alt F		Display Function Key Labels

AT0T 610                     - 4 -             Terminal Emulation

Unshifted and shifted combinations apply to the following keys on

the numeric keypad (These are also displayed on the second Help

screen accessed by ALT ?):

Keypad Key	Unshifted	Shifted

Alt 1		END

Alt Ctrl 1	CMD

Alt 2

Alt Ctrl 2	OPEN		CLOSE

Alt 3		BEG

Alt Ctrl 3	CANCL

Alt 4		PREV

Alt Ctrl 4	SUSP

Alt 5		PRINT

Alt Ctrl 5	RSUME

Alt 6		NEXT

Alt Ctrl 6	OPTS

Alt 7		HOME

Alt Ctrl 7	EXIT

Alt 8		CLEAR		REFRSH

Alt Ctrl 8	MSG

Alt 9		PAGE

Alt Ctrl 9	HELP

Use Alt F to display function key labels then Ctrl F-Key to

display root labels, or use Ctrl F-Key to directly display root

labels.  All roots are accessed by Ctrl F-Key.  Clusters refer to

each 8-column row of labels on the third Help screen accessed by

ALT ?.

Ctrl F1		Left Cluster 1

Ctrl F2		Left Cluster 2

Ctrl F3		Right Cluster 1

Ctrl F4		Right Cluster 2

Ctrl F5		Cursor Functions

Ctrl F6		Print Functions

Ctrl F7		User Functions

Ctrl F8		System Functions

Left Cluster 1 (Root 1):	Left Cluster 2 (Root 2):

CLEAR LINE			MOVE

RSTRT REF			COPY

CREAT				DLETE

SAVE				DLETE CHAR

UNDO				SLECT MARK

REDO				INPUT MODE

FIND

RPLAC

AT0T 610                     - 5 -             Terminal Emulation

Right Cluster 1 (Root 3):	Right Cluster 2 (Root 4):

EXIT				CANCL

MSG				PRINT

HELP				RFRSH

SUSPD				PAGE

RSUME				BEG

OPTS				END

CMD				PREV

CLOSE/OPEN			NEXT

Cursor Functions (Root 5):	Print Functions (Root 6):

HOME				PRINT ON-LINE

UP ARROW			STOP PRINT

DOWN ARROW

LEFT ARROW

RIGHT ARROW

CLEAR/RESET

User Functions (Root 7):	System Functions (Root 8):

F1-F8				F1-F8

Local Horizontal Scroll Keys:

Shift Ctrl Left Arrow or Right Arrow	Scroll Left or Right

Shift Ctrl Up Arrow or Down Arrow	Home Left or Right

AT0T 610                     - 6 -             Terminal Emulation