Hazeltine 1500

Introduction

The Hazeltine 1500 Video Display Terminal can be emulated using

Softerm by entering Hazeltine 1500 for the terminal emulation when

generating a configuration in terminal setup.  The Hazeltine 1500

general capabilities supported by Softerm include the full 128

character set with 80 X 24 display, full and half duplex modes,

scrolling, absolute cursor positioning, and special control

character sequences including all remote commands.

Features

The display format of the Hazeltine 1500 is 24 lines by 80

characters with the capability to display 94 of the 128 ASCII

character codes.  An Upper/Lower Case enable switch is also

provided allowing the user to select between upper case only or

upper and lower case display.  Softerm supports only upper and

lower case display.  Softerm provides support for the keyboard

functions ESC, DEL, RETURN, LINE FEED, BACK SPACE, TAB, BREAK,

RESET, HOME, CLEAR, Clear Foreground, Clear End of Line, and Clear

End of Screen.  The Clear functions are provided on separate

keyboard functions and the Right Arrow key is used to provide

cursor right and the Left Arrow key is used to provide cursor left

on keyboard.

The remote command features of the Hazeltine 1500 provide the user

with the capability to control the terminal via the host computer

software.  For the terminal to execute a remote command, the

command code must be preceded in some cases by a lead-in code

which is a tilde ($7E) character.  Softerm supports all remote

terminal commands and switch settings.

Hazeltine 1500 Keyboard Functions

The following table defines the standard keyboard functions for

the Hazeltine 1500 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Backspace		BS or DEL

Shift Backspace	DEL or BS

Home			Home Cursor

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

PgUp			Clear Screen

Ctrl PgUp		Clear Foreground

PgDn			Erase to End of Screen

End			Erase to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Hazeltine 1500               - 1 -             Terminal Emulation