IBM 3101 Model 1X

Introduction

The IBM 3101 Display Terminal Models 10, 12, or 13 can be emulated

using Softerm by selecting IBM 3101-1X for the terminal emulation

when generating a configuration in terminal setup.  Softerm

supports all the general capabilities of the IBM 3101 Model 1X

including Character Transmission, Full Duplex Operation, Half

Duplex Operation with full duplex communications, Auxiliary

Printer Interface, full 128 character set and 80 X 24 display,

Mono/Dual case, Program Function Keys, Keyboard lock/unlock,

Cursor control, Erase functions, Tab forward, Auto New Line,

Scroll On/Off, CR/CR-LF, Auto Line Feed, and Audible Alarm.

Features

The display format of the IBM 3101 Model 1X display terminal is 24

lines by 80 characters with the ability to display all 128 ASCII

character including 32 control codes in the Transparent Mode.  The

3101 also includes an operator information area at the bottom of

the display screen as an operation interface.  It displays various

modes and status of the 3101 communications and operations.

Softerm does not support the operator information area.

The IBM 3101 Model 1X functions only in the character mode.  Block

mode is available only on Models 20, 22, and 23.  Softerm supports

only the character mode features in the emulation of the 3101

Model 1X display terminal.  The full duplex operation mode and

half duplex operation mode when using a full duplex communications

facility are supported.  Half duplex operation using a half duplex

communications facility with or without reverse channel is not

supported.  Thus operation of the 3101 emulation with a 202-type

modem with switched RTS (request to send) is not supported.

The Softerm terminal emulation options Auto Wrap, Page Mode, and

Read Function Terminator correspond to IBM 3101 setup switch

options Automatic New Line, Scroll On/Off, and Line Turnaround

Character Selection respectively.

Setup switch options available on the IBM 3101 Model 1X and not

supported by Softerm include Mono character set selection,

Interface Selection, Modem Control, Reverse Channel Control, and

Reverse Video On/Off.

Softerm supports all the 3101 Model 1X keyboard functions.  The

AUX key capability is provided by the Softerm Ctrl PrtSc Printer

Enable/Disable Function.

Host-initiated functions which Softerm recognizes but ignores

include Logical Keyboard Lock and Set Control.

IBM 3101-1X                  - 1 -             Terminal Emulation

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� IBM 3101-1X Emulation Options ����Ŀ

Read Function Terminator: CR ($0D)

The emulation option Read Function Terminator allows the character

which terminates the locally initiated Program Function Key

sequences and the response to host initiated functions Read

Status, Read Switches and Read Cursor Position to be specified.

This corresponds to the Line Turnaround Character Selection

switches in the 3101 terminal.  The character selected can be CR

($0D), EOT ($04), ETX ($03), or XOFF ($13) depending the

requirements of the host computer.  If the XOFF character is

selected, the user should insure that the Terminal Emulation

Parameter Receive Pacing is set to None.

IBM 3101-1X Keyboard Functions

The following table defines the standard keyboard functions for

the IBM 3101 Model 1X terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Ctrl PrtSc		AUX

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt A			Send Answerback Message

Alt B			Break

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Home			Home

PgUp			Erase Input

Ctrl PgUp		Clear All

PgDn			Erase EOS

End			Erase EOL

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

F1	PF1		F5	PF5

F2	PF2		F6	PF6

F3	PF3		F7	PF7

F4	PF4		F8	PF8

IBM 3101-1X                  - 2 -             Terminal Emulation