TeleVideo 925

Introduction

The TeleVideo Model 925 terminal can be emulated using Softerm by

selecting TeleVideo 925 for the terminal emulation when generating

a configuration in terminal setup.  Softerm supports all the

general capabilities of the TeleVideo 925 including the full 128

character set and 80 X 24 display, video attributes, Tab and Back

Tab, editing features, cursor controls, conversational full or

half duplex transmission, block mode transmission, and auxiliary

port control including page print, buffered transparent, and

buffered with screen copy.

Features

The display format of the TeleVideo 925 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.  The

TeleVideo 925 also displays a 25th status line which is simulated

by Softerm.  The 925 has the option of one additional page of

display memory.  Softerm supports only a single page of display

memory.

Softerm supports all available keyboard functions.

Functional commands recognized and processed but ignored by

Softerm include Back Page, Advance Page, Keyclick On, Keyclick

Off, Auto Page On, Auto Page Off, and Load Time.

Softerm includes support for the Printer Port if the terminal

configuration includes a printer.  If either an Enable

Bidirectional Port, Extension Mode On, or Transparent Print On

remote command is received, all subsequent data is sent to the

printer until a Disable Bidirectional Port, Extension Mode Off, or

Transparent Print Off command is received.  In the transparent

mode data is sent only to the printer and not displayed, otherwise

the received data is both displayed and printed.  The printer port

may also be enabled by the Ctrl PrtSc keyboard function.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������ TeleVideo 925 Emulation Options �����Ŀ

Operating Mode: Character

Edit Mode: Duplex

Page Attributes: No

Send Key = Send Line: No

The emulation option Operating Mode selects the transmission mode

of the terminal and may be selected as either Character or Block.

In Block mode, characters entered from the keyboard are stored in

a buffer which may be corrected and edited by the operator, and

then transmitted to the host when the SEND key is pressed.

The emulation option Edit Mode sets the edit keys for local

operation only if Local is selected and the Duplex Edit Mode if

TeleVideo 925                - 1 -             Terminal Emulation

Duplex is entered.  In Duplex Edit Mode the edit keys will be

transmitted as alphanumeric keys and operated on according to the

current Full or Half Duplex mode.  The editing keys are TAB, BACK

TAB, CHAR INSERT, CHAR DELETE, LINE INSERT, LINE DELETE, LINE

ERASE, PAGE ERASE, BACKSPACE, HOME, CURSOR UP-DOWN-RIGHT-LEFT,

CLEAR SPACE, SEND, and PRINT.

The emulation option Page Attributes if set to Yes will allow the

Set Video Attribute command to set the video attributes from the

current cursor position to the end of the screen.  If this option

is set to No, the Set Video Attribute command affects only the

current line.

The emulation option Send Key = Send Line is used to activate the

SEND LINE key function when the SEND key is pressed.  If Yes is

selected, the SEND key will function as the SEND LINE key.  If No

is entered, the SEND key will function as the SEND PAGE key.

Operator Status Line Display

The TeleVideo 925 terminal emulation includes a status line which

may be displayed on the 25th line by pressing the Alt T key.  The

status information displayed includes Row and Column, Local or

Duplex edit mode, Character or Block mode, H.I.  (Half Intensity)

mode, Protect mode, and Keyboard Lock mode.

The Block or Character mode setting can be toggled while the

status is displayed by pressing the B key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking RIGHT ARROW appears in the first column

position.  For example, to toggle from the Block mode to the

Character mode, first display the status by entering Alt T and

then press the B key.  Pressing the E key will cause the Local or

Duplex editing mode to be toggled.  Pressing the H key will cause

the H.I.  mode to be toggled between On and Off.  Pressing the P

key will cause the Protect mode setting to be toggled between On

and Off.  Pressing the K key will cause the Keyboard Lock setting

to be toggled between On and Off.  Pressing the M key will toggle

Monitor mode On and Off.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

TeleVideo 925 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 925 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt Enter		SEND

Shift Enter		SEND LINE ALL

Ctrl Enter		SEND PAGE ALL

TeleVideo 925                - 2 -             Terminal Emulation

Ins			CHAR INSERT

Alt Ins		LINE INSERT

Ctrl Esc		LOC ESC

Shift PrtSc		PRINT

Ctrl PrtSc		Enable/Disable Printer Port

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Tab			TAB

Shift Tab		BACK TAB

Home			HOME

PgUp			CLEAR SPACE

Ctrl PgUp		CLEAR ALL (Nulls)

PgDn			PAGE ERASE (Spaces)

Ctrl PgDn		PAGE ERASE (Nulls)

End			LINE ERASE (Spaces)

Ctrl End		LINE ERASE (Nulls)

Del			CHAR DELETE

Alt Del		LINE DELETE

Backspace		BS or DEL

Shift Backspace	DEL or BS

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt A			Send Answerback Message

Alt B			Break

Alt F			FUNCT

Alt T			Display 925 Status Line

F1	F1	Shift F1	Shift F1

F2	F2	Shift F2	Shift F2

F3	F3	Shift F3	Shift F3

F4	F4	Shift F4	Shift F4

F5	F5	Shift F5	Shift F5

F6	F6	Shift F6	Shift F6

F7	F7	Shift F7	Shift F7

F8	F8	Shift F8	Shift F8

F9	F9	Shift F9	Shift F9

F10	F10	Shift F10	Shift F10

Alt F1	F11	Alt Shift F1	Shift F11

TeleVideo 925                - 3 -             Terminal Emulation