Perkin-Elmer 1251

Description

The Softerm emulation of the Perkin-Elmer 1251 terminal supports

all general capabilities of the actual terminal including the full

128-character ASCII set, including control characters in

transparent mode, 32 business form-drawing characters, 80 X 24

display, full 32 programmable function keys (including "HERE IS"

key), reverse screen image, online/offline, host/user addressable

status area, eight host-selectable print options, user/host

selectable conversational/block mode, user/host definable

configuration, and full Polled environment capablities.

Features

Softerm supports all remote terminal commands except: Lock Light

Pen, Unlock Light Pen, Blank Screen, Restore Screen, Set Attribute

Byte Light Pen Field, and Diagnostic Function.

Perkin-Elmer 1251 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����������� Perkin-Elmer 1251 Emulation Options �����Ŀ

������������������� Keyboard Control ����������������Ŀ

Esc,Edit:No  Cursor Pos:No  Tab,Backtab:No    CR,LF:No

F01-F04:No     F05-F08:No      F09-F12:No  F13-F16:No

�������������������� Keyboard Lock ������������������Ŀ

Esc:No  Cursor Pos:No  Tab,Backtab:No    CR,LF:No

F01-F04:No     F05-F08:No      F09-F12:No  F13-F16:No

The emulation option Keyboard Control allows you to designate

certain Keygroups as Control-Protected; that is, the Control key

must be depressed simultaneously with that key.  To toggle the

Keyboard Control fields to Yes, press the Enter key to move from

field to field, and use the Space bar to toggle each field.

Control-Protected applies to Shifted function key counterparts

also.

The emulation option Keyboard Lock allows you to designate certain

Keygroups as Locked Out; that is, the depression of that key

causes no action.  To toggle the Keyboard Lock fields to Yes,

press the Enter key to move from field to field, and use the

Space bar to toggle each field.  Locked Out applies to Shifted

function key counterparts also.

Perkin-Elmer 1251            - 1 -             Terminal Emulation

After the preceding options have been specified, press the

Alt-Enter key combination and the following additional screen

is displayed:

���� Perkin-Elmer 1251 Emulation Options ���Ŀ

Operating Mode: Character

Polled: No

Null Suppress Unformatted: No

Null Suppress Formatted: No

Send Key =: Page

Print Key =: Page

Scroll: No

Inverse Video: No

Auto Tab: No

Send Status Line: No

Display Status Line: Yes

Return = Send: No

Clear to Spaces: No

Send Position Code: No

Send Clear All: No

Sound KeyBoard Lock: Yes

The emulation option Operating Mode allows choosing between

Character and Block modes.  If Block mode is selected, characters

entered from the keyboard are not transmitted as they are entered.

Data may be entered and then transmitted using the Alt-Enter

key combination.  The keyboard is locked in Block mode.  In

Character mode, communications with the host are conducted on a

character-by-character basis.

If the emulation option Polled is set to Yes, characters are

neither sent to the host nor displayed on your screen until the

terminal is polled.  If this option is set to No, the terminal

does not respond to polling.

The emulation option Null Suppress Unformatted determines whether

or not null characters are sent when the screen is unformatted.

If Yes is selected, nulls are not sent.

The emulation option Null Suppress Formatted determines whether or

not null characters are sent when the screen is formatted.  If Yes

is selected, nulls are not sent.

The emulation option Send Key specifies whether the Send key

transmits a Page of data; a Line of data; or a Message.

The emulation option Print Key specifies whether the Print key

prints a Line, a Page, or a Message.

The emulation option Scroll specifies whether the screen is to be

scrolled up one line or wrap to the Home position when the cursor

is on the last column of last line of the screen.  If Yes is

selected, the display will scroll up one line.  If No is selected,

the cursor will move to the Home position.

Perkin-Elmer 1251            - 2 -             Terminal Emulation

The emulation option Inverse Video, if selected as Yes, creates a

light background with a dark foreground.  If No is selected, the

background will be dark and the foreground light.

The emulation option Auto Tab (Formatted Screen), if set to Yes,

causes the cursor to automatically skip over protected fields and

attribute characters.  If set to No, the operator is restricted to

tabbing past attibute characters to access fields.

The emulation option Send Status Line, if set to Yes, will cause

the Send key to transmit the data entered on the Status Line, if

the terminal is also in Block mode and if the cursor is in the

User Area of the Status line.  In Character mode, data is

transmitted character-by-character as it is entered.  If this

option is set to No, the Send key does not transmit data entered

in the User Area of the Status Line.

The emulation option Display Status Line, if set to Yes, will

cause the Status Line to be displayed with Status Line operations

operational.  If set to No, the Status Line will not be displayed

and its operations will not be operational.

The emulation option Return = Send (Block mode), if set to Yes,

causes the Return key to operate in the same manner as the Send

key.  If set to No, the Return key retains its functions.

The emulation option Clear to Spaces, if set to Yes, causes

terminal and screen memory to clear to spaces for appropriate

clearing operations.  If set to No, terminal and screen memory

will be cleared to nulls.

The emulation option Send Position Code, if set to Yes, will cause

the terminal to transmit the appropriate Multicode sequences to

the host automatically and move the cursor accordingly, when the

Cursor Position key is pressed.  If set to No, no automatic

Multicode sequences will be transmitted to the host when the

Cursor Position key is pressed.

If the emulation option Send Clear All (Edit Mode operation) is

set to Yes, the terminal will transmit the sequence Multicode R _

(underline character - hex 5F) Request to Send Terminator to the

host.  If the host responds with any read response, the screen is

cleared.  If the option is set to No, the screen is cleared upon

encountering the Clear All Edit Mode operation.

The emulation option Sound KeyBoard Lock, if set to Yes, sounds

the alarm when the keyboard is locked.  If set to No, the alarm is

disabled.

Perkin-Elmer 1251            - 3 -             Terminal Emulation

After the preceding options have been specified, press the

Alt-Enter key combination and the following additional screen

is displayed:

���� Perkin-Elmer 1251 Emulation Options ��Ŀ

��������������� Terminators ���������������Ŀ

Send Line:

Field:

Unformatted Line:

Unformatted Page:

Formatted Page:

Request To Send:

Print Line:

������������ Special Characters �����������Ŀ

Multicode Character: �ESC�

Poll Character:

Ring Bell: 0

Up to four ASCII characters (in a field of four) may be designated

for each communication terminator listed below.  Up to eight ASCII

characters (in a field of eight) can be designated for the Print

Line Terminator.

Softerm allows the user to input up to the maximum amount without

including the ASCII CAN character (018H) as the terminator, or if

the field is less than maximum, then the CAN character can be

included.  The ASCII CAN character is not included as part of the

terminator and is not transmitted.

The emulation option Send Line allows you to specify the

terminator to be appended automatically to each line for the Send

Line condition for both formatted and unformatted screens.  This

terminator is appended automatically to the transmission of a

message of a function key (except the function key designated as

the HERE IS key) that has been defined in configuration.

The emulation option Field allows you to specify the terminator

which is appended automatically to each field on the transmission

of formatted screens.

The emulation option Unformatted Line allows you to specify the

terminator which is appended automatically to each line for the

Send Message and Send Page conditions on the transmission of

unformatted screens.

The emulation option Unformatted Page allows you to specify the

terminator which is appended automatically to each message or page

for for the Send Message and Send Page conditions on the

transmission of unformatted screens.

The emulation option Formatted Page allows you to specify the

terminator which is appended automatically to each message or page

for the Send Message and Send Page conditions on the transmission

of formatted screens.

Perkin-Elmer 1251            - 4 -             Terminal Emulation

The emulation option Request to Send allows you to specify the

terminator which is appended automatically to each transmission of

a Request to Send Multicode sequence.  This includes a

transmission as a result of a function key that has not been

defined in configuration.

The emulation option Print Line allows you to specify the

terminator which is appended automatically to each line of data

that is transmitted through the terminal's printer port.

The following fields are designated by a single ASCII character:

The emulation option Multicode Character allows you to specify the

ASCII character which will be used to enable special functions.

This character can be in the range 00H to 07EH.  It defaults to

the Esc character (hex 1B).

The emulation option Poll Character allows you to specify the

ASCII character which will be used as the Polling/Selection

address character to distinguish one terminal from another if your

terminal is used in a system of polled terminals.

The emulation option Ring Bell allows you to specify the column

number where the bell will be rung when the cursor reaches the

column.  This number can be in the range 0 to 80 and defaults to

column 0.

After the preceding options have been specified, press the

Alt-Enter key combination and the following additional screen

is displayed:

�������� Perkin-Elmer 1251 Function Key Definitions ������Ŀ

Here Is = F01

�NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

�NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

�NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

�NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

�NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

Note: Each line actually has 16 �NUL� indicators.  The screen

was edited to allow it to fit on the page.

The Perkin-Elmer 1251 emulation supports 16 function keys

(shiftable to 32), including designation for the "HERE IS" key.

Function keys are redefined by inputing the following sequence

into the 400-byte buffer on the last setup screen:

1. Press Enter to highlight the first row.

2. Enter a capital F

3. Enter the 2-digit number of the function key (leading

zeroes must be entered -- ie: F02)

4. Enter the Equals character, =

5. Input the message

6. Terminate with the ASCII Cancel character (Control X)

Example: F02=This is function key two(Control X)

Perkin-Elmer 1251            - 5 -             Terminal Emulation

The Cancel character is not part of the message and is therefore

not transmitted.

Function keys F01, F02, F03 and F04 send their default messages in

EDIT mode.

When undefined, the Function keys transmit a Request to

Send/Attention Identifier Character/Request to Send Terminator

sequence to the host.

When defined in configuration, function keys F13, F14, F15 and

F16, and their shifted counterparts F29, F30, F31 and F32 do not

transmit their respective messages to the host in Block mode;

rather their messages are displayed on the screen.  In

Conversational mode, the message is transmitted.

Perkin-Elmer 1251 Operator Status Line Display

The Perkin-Elmer 1251 terminal emulation includes a status line

which may be displayed on the 25th line by pressing the ALT T key,

if Display Status Line is set to Yes in the second terminal-

specific configuration screen.

The status information and indicators which may be displayed

include:

Column		    C--

Online		    LN

Local		    LCL

Operating mode

Block		   BL

Character	   CV

Program	   PG

Edit		   Edit

Insert Character    Insert

Immediate	    IM

Send All	    AL

Send Unprotected    UN

Send Modified       M

Request to Send     RS

Numeric Characters

Only Field  NUMRC FLD

Protected Field     PROTCT FLD

Keyboard Locked     KBD LOCKED

Pressing ALT T again will remove the status line display.

Perkin-Elmer 1251            - 6 -             Terminal Emulation

Perkin-Elmer 1251 Keyboard Functions

The following table defines the standard keyboard functions for

the Perkin-Elmer 1251 terminal emulation:

Keyboard		Function

Enter			CR

Alt Enter		Send

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

BackSpc		Backspace

Shift BackSpc		Delete

Tab			TAB

Shift Tab		BACK TAB

Home			HOME

Alt B			BREAK

Alt E			Enter/Exit Edit Mode

Alt R			Error Reset

F1 - F10		Function Keys 1-10

Use Alt with the following keys:

F1 - F6	Function Keys 11 - 16

Use Shift with the following keys:

F1 - F10	Function Keys 17 - 26

Use Alt Shift with the following keys:

F1 - F6	Function Keys 27 - 32

Edit Mode Keys:

F05		PAGE

F06		Block/Conversational Mode

F07		Insert Mode

F08		Insert Line

F09		Delete Character

F10		Delete Line

F11		Clear L/F

F12		Clear Unprotected

F18		Program Mode

F19		Transparent Mode

F20		Line/Local

F22		Clear Tabs

F23		Set Tab

F24		Clear All

F26		Auto Linefeed

F27		New Line

F28		Scroll Enable

Perkin-Elmer 1251            - 7 -             Terminal Emulation