Tandberg 2200 Video Display Terminal

Introduction

The Tandberg 2200 Terminal can be emulated using Softerm by

selecting Tandberg 2200 for the terminal emulation when generating

a configuration in terminal setup.  The Tandberg 2200 general

capabilities supported by Softerm include the full 128 character

ASCII set with 80 X 25 display; asynchronous character

transmission mode; visual attributes; full keyboard emulation; and

auxiliary port printer support; Host and user programmable

function keys (PUSH); Multiple language sets; Special line drawing

set and graphics characters.

Important:  If the Terminal Parameters Display Parameters Extended

Text option is active, the Code Page setting used by the Multi-

National Character Translation program will be disabled.  Only the

United States Code Page will be available.  Note:  All other MCT

options will be effective.

Features

Host-initiated Tandberg 2200 operational commands which Softerm

recognizes and processes include Sound Alarm, Backspace, Linefeed,

Vertical Cursor Positioning, Carriage Return, Horizontal Cursor

Positioning, Write Foreground Characters, Write Background

Characters, Clear Screen, Cursor Home, Field Tab, Direct Cursor

Address, Send Cursor Address, Enable Auxiliary Output with

Display, Enable Auxiliary Output without Display, Disable

Auxiliary Output, Programmable Function Keys (PUSH).

The Tandberg 2200/9 emulation recognizes but ignores the following

switches: ATTRIBUTE; and Underline Representations Inv/Und/Low,

Blnk/Und/Inv and Und/Blnk.

Tandberg 2200 Emulation Options

After the last standard Terminal Parameters menu is displayed, the

following menu will appear:

���� TANDBERG TDV 2200/9 Emulation Options ��Ŀ

Graphics Rendition: ATTRIBUTE

Print Extent: Full Screen

Print Terminator: None

Extended Control: On

Screen Display: Normal

Scroll Rate: Jump

Keyboard Set: Norwegian

Margin Bell: Yes

BREAK Enabled: Yes

Underline Representation: Underline

Vertical Editing Mode: Following

Beginning of Line Wrap: Stop

End of Line Wrap: Stop

Roll/Page Mode: Roll

Tandberg TDV 2200            - 1 -             Terminal Emulation

The emulation option Operating Mode may be toggled to ATTRIBUTE,

UNDERLINE or SGR.  If set to Underline, the Underline

Representation setting (described below) will be in effect.  If

set to SGR, graphics character sets 2 and 3 will be available, so

long as the PC has an installed graphic adapter board and Softerm

has been installed with the Extended Text option.  The ATTRIBUTE

setting is ignored and has no effect.

The Print Extent option determines what portion of the screen is

printed when the Shift PrtSc keyboard function is executed or a

print screen command is received.  If this option is specified as

Full Screen, the full screen will be printed.  If this option is

specified as Scrolling Region, only the area defined in the

current scrolling region will be printed.

The Print Terminator option specifies if a Formfeed character is

generated after the screen is printed using the Shift PrtSc

keyboard function or when a print screen command is received.  If

this option is specified as Form Feed, a form feed character ($0C)

is appended to the data sent to the printer by the print screen

function.  If this option is specified as None, no form feed is

included.

The emulation option Extended Control may be toggled to On or Off.

If set to On, the Control Sequence Introducer (Esc [) is in

effect.  If set to No, the CSI escape sequence has no effect.

The Screen Display option allows the initial screen display mode

to be selected as Normal or Reverse.  If Normal is selected, the

foreground and background screen colors are displayed normally.

If Reverse is selected, the foreground and background colors are

reversed.

The Scroll Rate option allows the scroll rate to be selected.  If

Jump is selected, data will be scrolled at a fast jumpy rate.  If

6 Lines/Second is selected, smooth scrolling is simulated by

scrolling at a slower rate.

The emulation option Char Set may be toggled to Int English,

Norwegian, German, or Swedish.  This option is effective only if a

graphics adapter card is installed in the PC and the user has

installed Softerm with the Extended Text option.  The character

sets then will correspond to the International, Norwegian, Swedish

and German sets listed in the Tandberg TDV 2215 Functional

Specifications manual for Firmware Revision Level 11.

The Margin Bell option selects whether the audible alarm is

sounded as characters are keyed when approaching the right margin.

If this option is set to Yes, the audible alarm is sounded when a

character is entered at column 74.  If this option is set to No,

no alarm is issued.

The BREAK Enabled option determines if the BREAK key Alt B is

enabled.  If this option is set to No, the BREAK function is

disabled.

Tandberg TDV 2200            - 2 -             Terminal Emulation

The option Keypad Mode allows the default operating mode of the

numeric keypad to be selected.  If set to Numeric and NumLock is

On, the keypad will generate numbers.  If NumLock is Off, the

keypad will generate Local Edit Functions.  If set to Application

and NumLock is On, the special keypad applications mode is enabled

and generates special code sequences.  If NumLock is Off, the

keypad will generate Local Edit Functions.  Note: When used with

the Alt key, keypad keys always generate an Application Mode

sequence.

The emulation option Underline Representation lets the user select

the underline graphic rendition.  However, the effect of this

setting depends on the type of graphic adapter board installed in

the PC.  The available settings and their effective modes are:

Underline:	All modes

Normal:		MDA (text) only

Inverse:	All modes

Low:		All modes

Inv/Low:	All modes

Und/Inv:	Hercules and EGA only

Und/Low:	All modes

Blink:		MDA and EGA only

Inv/Blnk:	MDA and EGA only

Low/Blnk:	MDA and EGA only

Inv/Low/Blnk:	MDA only

The emulation option Vertical Editing Mode may be toggled to

Following or Preceding.  If set to Following, an Insert Line

command will push following lines down.  If set to Preceding, an

Insert Line command will push preceding lines up.

The emulation option Horizontal Scroll Delay determines the amount

of time in one tenth second increments that Softerm PC will delay

before horizontal scrolling is initiated when no characters have

been received for the amount of time specified by the delay.  This

option prevents unnecessary shifting of the screen when a

continuous stream of characters is being received.

The emulation option Beginning of Line Wrap may be toggled to Stop

or Wrap and controls the cursor movement when the cursor is in the

first position of a line.  If set to Stop, a movement command to

the left will have no effect.  If set to Wrap, a movement command

to the left, such as a BackSpace, will wrap the cursor around to

the last position of the preceding line.  This has no effect,

however, if the cursor is in the Home position.

The emulation option End of Line Wrap may be toggled to Wrap or

Stop and controls the cursor movement when the cursor is in the

last position of a line.  If set to Wrap, a cursor movement

command to the right, such as a CAN (18 Hex) code, will move the

cursor to the first position of the following line.  If set to

Stop, a cursor movement command to the right will have no effect,

and the cursor will remain in the last position on the line.

Further characters will overwrite preceding characters.

After the preceding options have been specified, press the Alt

Enter key and the following additional option screen is displayed:

Tandberg TDV 2200            - 3 -             Terminal Emulation

������ TANDBERG TDV 2200/9 Emulation Options �����Ŀ

Enable Character Transmit Delay: No

Character Delay (1/10 Secs.): 1

�������������������� Tab Stops ��������������������

Columns          T        T        T        T

1-40      1234567890123456789012345678901234567890

Columns   T      T        T        T        T

41-80     1234567890123456789012345678901234567890

Softerm PC allows default tab stops to be defined which are set at

initialization, when Ctrl Break is pressed, or any control

sequence is received which would return the terminal to its

initial state.  The tab format consists of 2 string entry fields,

for columns 1-40 and 41-80.  Entering any non-blank character,

such as a T, will result in a tab stop being set at the current

location.

After the preceding options have been specified, press the Alt

Enter key and the following additional option screen is displayed:

��������� TDV 2215 �� Push Key Programming ������Ŀ

P1: �NUL��NUL�

P2: �NUL��NUL�

P3: �NUL��NUL�

P4: �NUL��NUL�

P5: �NUL��NUL�

P6: �NUL��NUL�

P7: �NUL��NUL�

P8: �NUL��NUL�

P9: �NUL��NUL��NUL��NUL��NUL��NUL�

P10: �NUL��NUL��NUL��NUL��NUL��NUL�

P11: �NUL��NUL��NUL��NUL��NUL��NUL�

P12: �NUL��NUL��NUL��NUL��NUL��NUL�

P13: �NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

P14: �NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

P15: �NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

P16: �NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL��NUL�

Each function key can be assigned a string containing a maximum of

48 characters.  The string will be played back when the function

key is pressed.  For instance, you could program function keys F1

and F2 to output the following:

F1: Softronics, Inc.�cM��cJ�5085 List Dr.�cM��cJ�Colorado Springs,

CO 80919�cM��cJ�.  The �cM� and �cJ� represent the carriage return

and line feed sequences which are inserted into the string by

pressing Ctrl-M and Ctrl-J.  When you press F1 and F2, the screen

output will resemble:

Softronics, Inc.

5085 List Dr.

Colorado Springs, CO 80919

Tandberg TDV 2200            - 4 -             Terminal Emulation

Note1: To insert the Escape character into the string, press the

Esc key twice.

Note2: The string must end with either a NUL (which may be

inserted by pressing Ctrl @) or the entire 48 characters must be

used.  If the string contains fewer than 48 characters and does

not end with a NUL, the string becomes padded with spaces, causing

unpredictable results.

Tandberg 2200 Keyboard Functions

The following table defines the standard keyboard functions for

the Tandberg 2200 terminal emulation:

Keyboard		Function

Enter			CR, CR/LF

Shift Enter		Shift Return

Backspace		BS

Tab			Next Foreground Field

Home			Home

PrtSc			Enable/Disable Printer

Alt			Line Feed

Ctrl Break		Soft Reset

F1	PUSH string 1	Shift F1  PUSH string 11

F2	PUSH string 2	Shift F2  PUSH string 12

F3	PUSH string 3	Shift F3  PUSH string 13

F4	PUSH string 4	Shift F4  PUSH string 14

F5	PUSH string 5	Shift F5  PUSH string 15

F6	PUSH string 6	Shift F6  PUSH string 16

F7	PUSH string 7

F8	PUSH string 8

F9	PUSH string 9

F10	PUSH string 10

Alt F1		SI

Alt F2		SO

Alt F3		Delete Line

Alt F4		Insert Line

Alt F5		Delete Character

Alt F6		Insert Character

Alt F7		Erase Line

Alt F8		Erase Page

Alt F9		CSI

Alt F10	CLEAR (Host Message)

Tandberg TDV 2200            - 5 -             Terminal Emulation