TeleVideo 955 Display Terminal

Description

The Televideo 955 display terminal can be emulated using Softerm

by selecting Televideo 955 for the terminal emulation when

generating a configuration in terminal setup.  The Televideo 955

general capabilities supported by Softerm include the full 128-

character ASCII set; international, graphics, and multinational

character sets; 80- or 132-column screen width; 96 lines (4 pages

display memory); 2 sets of function keys, 64 function keys total;

transparent, copy, and bidirectional print modes; addressable

status message area; addressable user message areas 1 and 2; and

individual locked lines.

Features

The display format for the TeleVideo 955 is host or user

selectable 80 columns or 132 columns by 96 lines.  Individual

pages can be created (up to 4) as either 1 page of 96 lines, 2

pages of 48 lines, or 4 pages of 24 lines.  The host or user may

select a dark or light background, normal intensity (dim or

normal), and one of 8 supported international character: United

States, United Kingdom, French, German, Spanish, Finnish,

Norwegian, and Italian.  Status line attributes are host/user

selectable, including: reverse, normal, underline, and reverse-

underline.  Operating mode is also host/selectable as either

Televideo 955 compatibility mode or Televideo 950 compatibility in

either Block or Conversational modes.

Softerm supports all standard and shifted keys on the Televideo

955.

Softerm supports all commands with the following exceptions, which

it recogniaes but does not support: Run the Self Test, Select

Screen Refresh Rate, Execute a User-Installed Program, Lock/Unlock

SET UP Key, Lock/Unlock BREAK Key, Keyboard Repeat On/Off,

Keyclick On/Off, Select Screen Saver Time, Turn Screen On/Off,

Select Printer Port Operating Values, Select the Receive Buffer

Fill Limit, Select the Character Transmit Delay Rate, and Select

the Contents of the Top Information Line.

TeleVideo 955 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

TeleVideo 955                - 1 -             Terminal Emulation

������� Televideo 955 Emulation Options �����Ŀ

Compatibility Mode: 955

Operating Mode: Character

Columns: 80

Wrap: Yes

Background: Dark

Attribute: Space

Base: Page

Normal: Full

Status: Reverse

Character Set: United States

CR = CR

Scroll Rate: Jump

Horizontal Scroll Delay (1/10 Secs.):2

Local Horizontal Scroll Increment:1

The emulation option Compatibility Mode may be set to 955

(default) or 950.  If set to 955, the terminal operates as a

TeleVideo 955 terminal as described in this supplement.  If set to

950, the terminal correctly responds to TeleVideo 950 commands.

The emulation option Operating Mode selects the transmission mode

of the terminal and may be set to Character or Block.  If set to

Block, data in the specified area is sent to the host system.

This mode allows the operator to edit the data before sending it

to the host system.  If set to Character, data entered from the

keyboard is simultaneously sent to the host system and displayed

on screen.

The emulation option Columns may be set to 80 or 132 and defines

the number of columns that may appear on each line.  The TeleVideo

955 terminal emulation emulates 132-column mode by using

horizontal scrolling.

If the emulation option Wrap is set to Yes, the cursor wraps to

the start of the next line after reaching the last column of the

current line.  If set to No, the cursor does not wrap to the next

line.

If the emulation option Background is set to Dark, the screen

background will be dark with light characters.  If set to Lite,

the screen background will appear light with dark characters.

The emulation option Attribute may be set to Space or No Space.

If set to Space, visual attributes occupy a space (are embedded)

on the screen.  If set to No Space, visual attributes are not

embedded on the screen and do not occupy a space.

If the emulation option Base is set to Page (the default), visual

attributes affect data from the cursor to the end of the page.  If

set to Line, visual attributes affect data from the cursor to the

end of the line.

TeleVideo 955                - 2 -             Terminal Emulation

If the emulation option Normal is set to Full, the normal screen

intensity is full and write protected fields appear in half

intensity.  If set to Half, the normal screen intensity is half

and write protected fields appear in full intensity.

The emulation option Status may be set to Normal, Reverse,

Underline or Reverse-Underline and defines the visual attribute

used to display the status line.

The emulation option Character Set may be set to reflect in which

character set certain international characters are to be

displayed.  Softerm supports the following 8 international

character sets: United States, United Kingdom, French, German,

Spanish, Finnish, Norwegian, and Italian.  For instance, if United

Kingdom is selected, the # character will be displayed as .

The emulation option CR= may be set to CR, in which case the ENTER

key sends only a Carriage Return code, or CR, LF, in which case

the ENTER key sends both the Carriage Return and Line Feed codes.

The emulation option Scroll Rate may be set to Jump, Normal, Slow

or Fast.  If set to Jump, data scrolls onto the screen at the same

rate it comes from the host.  If set to Normal, the scroll rate is

4 lines per second.  If set to Slow, the scroll rate is 2 lines

per second, and if set to Fast, the scroll rate is 8 lines per

second.

The emulation option Horizontal Scroll Delay (1/10 Secs.)  is the

delay time in tenths of a second that the horizontal scroll will

delay when the cursor reaches the right or left margin in 132-

column mode.

The emulation option Local Horizontal Scroll Increment determines

how many columns the screen will scroll horizontally when the

Softerm local scroll and home functions are used in 132-column

mode.

After the preceding options have been specified, press the Alt

Enter key combination and the following additional screen is

displayed:

������� Televideo 955 Emulation Options ����Ŀ

Down: Ctrl V (016H)

Editing Keys: Duplex

Enable Margin Bell: No

Enable Clear Space: Yes

Enable Escape Key: Yes

Function Key Save: Yes

Function Key Set: 1

Status Line: Stat

Monitor Mode: No

Print Mode: No

Flip: Yes

Answerback Message: �NUL��NUL��NUL�

TeleVideo 955                - 3 -             Terminal Emulation

If the emulation option Down is set to Ctrl V (016H), the DOWN key

(down arrow) sends a cursor down command.  If set to Ctrl J (0AH),

the DOWN key sends a line feed command.

The emulation option Editing Keys may be set to Local or Duplex.

If set to Duplex, the editing key codes go to the host in full or

half duplex modes.  If set to Local, the editing key codes go only

to the terminal, even in conversational modes.

If the emulation option Enable Margin Bell is set to Yes, the Bell

sounds when the cursor reaches the margin bell column when

entering keyboard data.  If set to No, the margin bell does not

sound.

If the emulation option Enable Clear Space is set to Yes, the

CLEAR SPACE key functions.  If set to No, the CLEAR SPACE key is

disabled.

The emulation option Enable Escape Key may be set to Yes or No,

and either enables or disables the ESC key.

The emulation option Function Key Save may be set to Yes or No.

If set to Yes, the contents of the function keys are stored in

non-volatile (permanent) memory.  If set to No, the keys return to

factory default values when you turn off the power.

The emulation option Function Key Set may be set to either 1 or 2

and selects either function key set one or two.

The emulation option Status Line may be set to No, Stat, User Line

1 or User Line 2.  If set to No, no status line is shown on the

last screen row.  If set to Stat, the TeleVideo 955 Status Line is

displayed on the last screen row.  Setting this option to User

Line 1 or User Line 2 causes User Message 1 or User Message 2 to

be displayed on the last screen row.  Note: This option sets the

default display.  Any of these options may be changed from the

terminal mode by pressing Alt T and choosing the desired option.

If the emulation option Monitor Mode is set to Yes, the terminal

displays command characters (control characters and excape

sequences) on the screen instead of acting on them.  If set to No,

the terminal processes commands normally.

The emulation option Print Mode may be set to No, Copy,

Transparent, or Bi-Directional.  If set to No, there is no print

mode.  If set to Copy, the terminal sends (copies) all data from

the host to the printer, as well as displaying it on the screen

and acting on any commands in the data stream.  Main and printer

port baud rates may differ.  If set to Transparent, the terminal

does not display data on its screen that flows from the main

through the printer port.  Existing screen data is undisturbed.

Main and printer port baud rates may differ.  If set to Bi-

Directional, the printer function operates the same as in Copy

mode.

If the emulation option Flip is set to Yes, the screen flips to

the next page of memory and continues updating when the terminal

receives a page print command.  If set to No, the screen stops

TeleVideo 955                - 4 -             Terminal Emulation

updating when the terminal receives a page print command, and the

current page remains displayed.

The emulation option Answerback Message lets you enter a 16-byte

(character) answerback message.

TeleVideo 955 Operator Status Line Display

The TeleVideo 955 terminal emulation includes a Status Line which

may be displayed on the 25th line by pressing Alt T.

The status information displayed includes the current active Page,

Row, Column, Duplex setting (FDX for Full Duplex and HDX for Half

Duplex), Line or Page mode, Protect mode if set, Keyboard Lock

status if set, Block mode indicator (BLK for Block or the current

duplex setting indicating the terminal is in Character mode), and

the Print Mode setting if that option has been set to other than

No in the Terminal Options menu.  The number of columns and the

Monitor Mode status do not have a Status Line indicator.

As shown in the Status Line menu, several settings may be toggled

to their opposite values without returning to the Terminal Options

menus.  In addition, pressing either 1 or 2 will clear the Status

Line menu, leaving either the User Message 1 or User Message 2

line at the bottom of the screen.

You may exit the Status Line menu by pressing either Alt T or Alt

Esc.  Alt T will clear the menu and leave the TeleVideo 955 Status

Line on the last screen row, while pressing Alt Esc will clear

both the menu and the Status Line.

TeleVideo 955 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 955 terminal emulation:

Keyboard		Function

Enter			Return

Shift Enter		Enter

Alt Enter		Send

Alt Shift Enter	Shift Enter

Ctrl Alt Enter		Shift Send

Home			Cursor Home

Ctrl Home		Shift Cursor Home

Right Arrow		Cursor Right

Ctrl Right Arrow	Shift Cursor Right

Left Arrow		Cursor Left

Ctrl Left Arrow	Shift Cursor Left

Up Arrow		Cursor Up

Ctrl Up Arrow		Shift Cursor Up

Down Arrow		Cursor Down

Ctrl Down Arrow	Shift Cursor Down

Tab			Tab

Shift Tab		Shift Tab

Alt Tab		Keypad Tab

Alt Shift Tab		Shift Keypad Tab

Ctrl Tab		Back Tab

TeleVideo 955                - 5 -             Terminal Emulation

Shift Ctrl Tab		Shift Back Tab

Ins			Character Insert

Alt Ins		Line Insert

Ctrl Ins		Shift Char Insert

Del			Character Delete

Ctrl Del		Shift Char Delete

Alt Del		Line Delete

Alt Ctrl Ins		Shift Line Insert

Alt Ctrl Num -		Shift Line Delete

PgUp			Clear Space

Ctrl PgUp		Shift Clear Space

PgDn			Page Erase

Ctrl PgDn		Shift Page Erase

Alt PgDn		Clear Entry (CE)

Alt Ctrl PgDn		Shift Clear Entry

End			Line Erase

Ctrl End		Shift Line Erase

Alt F			Funct

Alt T			Display TV955 Status Line

Shift PrtSc		Print

Ctrl PrtSc		Shift Print

Alt Num +		Page

Ctrl Num +		Shift Page

Ctrl NumLock		Hold Screen

Shift Esc		Local Escape

Use Shift with the following keys:

F1 - F10	Function Keys 1 - 10

Alt F1 - F6	Function Keys 11 - 16

Local Horizontal Scroll Keys:

Alt Ctrl Left or Right Arrow	Scroll Left or Right

Alt Ctrl Up or  Down Arrow	Home Left or Right

TeleVideo 955                - 6 -             Terminal Emulation