Lear Siegler ADM-5

Introduction

The Lear Siegler ADM-5 Video Display Unit can be emulated using

Softerm by selecting Lear Siegler ADM-5 for the terminal emulation

filename when generating a configuration in terminal setup.  The

ADM-5 general capabilities supported by Softerm include the full

128 character set with 80 X 24 display, full and half duplex

modes, scrolling, absolute cursor positioning, selectable auto new

line, editing functions, gated extension port operation, and

special control character sequences.

Features

The display format of the ADM-5 is 24 lines by 80 characters with

standard absolute cursor positioning with the home position in the

upper left of the display.  Softerm includes support for the

cursor control keys, the functional command keys, and also

provides support for the edit keys Erase to End of Line, and Erase

to End of Page.  Conversation modes of full or half duplex are

supported.

Switch selectable settings and features in the ADM-5 not supported

by Softerm include Secondary Channel, Code Turnaround and 202

Modem.  Softerm includes support for the optional feature

Automatic Answerback.

Softerm also includes support for the Gated Extension Port

Operation if the terminal configuration includes a printer.  If

the gated extension port option is specified, when an enable

extension port function code $0E is received from the host

computer, all subsequent data is sent to the printer until a

disable extension port function code $0F is received from the host

computer or the Ctrl Break key is pressed.  Data received is also

displayed on the video display.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

���� Lear-Siegler ADM-5 Emulation Options ��Ŀ

Gated Extension Port Operation: No

If this option is specified as Yes, the gated extension port

operation described previously is used.  If this option is

specified as No, the keyboard lock/unlock feature is enabled.

When a $0E function code is received, the keyboard is unlocked,

and when a $0F function code is received, the keyboard is locked.

Lear Siegler ADM-5           - 1 -             Terminal Emulation

Lear Siegler ADM-5 Keyboard Functions

The following table defines the standard keyboard functions for

the Lear Siegler ADM-5 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt A			Send Answerback Message

Alt B			Break

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Backspace		BS

Shift Backspace	DEL

Home			Home Cursor

PgUp			Clear Screen

PgDn			Erase to End of Screen

End			Erase to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		No Scroll

Ctrl PrtSc		Enable/Disable Extension Port

Lear Siegler ADM-5           - 2 -             Terminal Emulation