Hazeltine 1421 Video Display Terminal

Introduction

The Hazeltine 1421 Video Display Terminal can be emulated using

Softerm by selecting HAZ1421 for the terminal emulation when

generating a configuration in terminal setup.  The Hazeltine 1421

general capabilities supported by Softerm include the full 128-

character ASCII set with 80X24 display; asynchronous character

transmission mode; visual attributes; full keyboard emulation; and

auziliary port printer support.

Features

Softerm recognizes and processes all Hazeltine operational

commands.  However, the Lear Siegler ADM-3A and ADDS Consul 580

terminal emulations are not supported.

After the last standard screen in the Terminal Parameters mode in

Terminal Setup, the following screen will be displayed:

������ Hazeltine 1421 Emulation Options ����Ŀ

Auto LF: No

Wrap Around: No

Lead In: ESC

Font: UPPER CASE

The emulation option Auto LF (Automatic Line Feed) may be toggled

to No or Yes.  If set to Yes, all received carriage returns

automatically cause the cursor to move to the first character

position of the next line.  If the cursor was on the last

displahable character row, carriage returns will scroll the

display.  If set to No, all received carriage returns move the

cursor to the first character position of the same line.  Received

line feed characters will move the cursor down one line.  If the

cursor was on the last displayable character row, line feed

characters cause the display to scroll.

The emulation option Wrap Around may be toggled to Yes or No.  If

set to Yes, the cursor moves to the first column of the next row

without requiring a carriage return or line feed, whenever a

character is entered in the last column of a row, either from the

keyboard or as received data.  If the cursor was on the last row,

the display scrolls up.  If set to No, the cursor will remain at

the last column position of the row.  If data continues to be

entered or received, the characters overwrite the last displayed

character on the row.

The emulation option Lead In may be toggled to ESC or Tilde to

select the remote command lead in character.

The emulation option Font may be toggled to U/L CASE or UPPER

CASE.  If set to U/L CASE, the full 128-character ASCII set is

available for transmission and the 95-character alpha/numeric set

is available for display.  The Caps Lock key may be used for all

upper-case operation.  If set to UPPER CASE, all lower-case alpha

characters generated from the keyboard are converted to upper-case

Hazeltine 1421               - 1 -             Terminal Emulation

for transmission and display and all received characters are

converted to upper-case for display.  The Caps Lock key is

disabled.

Hazeltine 1421 Keyboard Functions

The following table defines the standard keyboard functions for

the Hazeltine 1421 terminal emulation:

Keyboard		Function

Enter			CR, CR/LF

Home			Home

Backspace		BS

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt Down Arrow		Line Feed

Tab			Next Foreground Field

PrtSc			Enable/Disable Printer

PgUp			Clear Screen

Ctrl PgUp		Clear Foreground

Alt B     Break

Hazeltine 1421               - 2 -             Terminal Emulation