TeleVideo 950

Introduction

The TeleVideo Model 950 terminal can be emulated using Softerm by

selecting TeleVideo 950 for the terminal emulation filename when

generating a configuration in terminal setup.  Softerm supports

all the general capabilities of the TeleVideo 950 including the

full 128 character set and 80 X 24 display, screen attributes,

cursor attributes, cursor control, editing, programmable function

keys, status line, conversational full or half duplex

transmission, block mode transmission, and auxiliary port control

including formatted or unformatted page print, buffered

transparent, and buffered with screen copy.

Features

The display format of the TeleVideo 950 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.  The

TeleVideo 950 includes 15 special graphic characters and a 25th

status line which are supported by Softerm.  The 950 has the

option of 1-3 additional pages (24 lines) of display memory.

Softerm supports only a single page (24 lines) of display memory.

Softerm supports all available keyboard functions.

Softerm recognizes and processes all functional commands with the

following exceptions:

The Linelock Line command will cause the current and all preceding

lines to become locked by setting the top margin to the current

line plus 1.  The Send Status Line Command will cause only a

carriage return to be transmitted.

Functional commands recognized and processed but ignored by

Softerm include Smooth Scroll, Normal Scroll, Back Page, Advance

Page, Keyclick On, Keyclick Off, Auto Page On, Auto Page Off, Page

Size, Screen On, Screen Off, Go To User ROM, Program Printer Port,

and Program CPU Port.

Softerm includes support for the Printer Port if the terminal

configuration includes a printer.  If either an Enable

Bidirectional Port, Copy (Extension) Mode On, or Transparent Print

On remote command is received, all subsequent data is sent to the

printer until a Disable Bidirectional Port, Copy (Extension) Mode

Off, or Transparent Print Off command is received.  In the

transparent mode data is sent only to the printer and not

displayed, otherwise the received data is both displayed and

printed.  The printer port may also be enabled by the Ctrl PrtSc

key.

TeleVideo 950                - 1 -             Terminal Emulation

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� TeleVideo 950 Emulation Options ���Ŀ

Operating Mode: Character

Edit Mode: Duplex

Page Attributes: No

The emulation option Operating Mode selects the transmission mode

of the terminal and may be selected as either Character or Block.

In Block mode, characters entered from the keyboard are stored in

a buffer which may be corrected and edited by the operator, and

then transmitted to the host when the SEND key is pressed.

The emulation option Edit Mode sets the edit keys for local

operation only if Local is selected and the Duplex Edit Mode if

Duplex is entered.  In Duplex Edit Mode the edit keys will be

transmitted as alphanumeric keys and operated on according to the

current Full or Half Duplex mode.  The editing keys are TAB, BACK

TAB, CHAR INSERT, CHAR DELETE, LINE INSERT, LINE DELETE, LINE

ERASE, PAGE ERASE, BACKSPACE, HOME, CURSOR UP-DOWN-RIGHT-LEFT,

CLEAR SPACE, SEND, and PRINT.

The emulation option Page Attributes if set to Yes will allow the

Set Video Attribute command to set the video attributes from the

current cursor position to the end of the screen.  If this option

is set to No, the Set Video Attribute command affects only the

current line.

Operator Status Line Display

The TeleVideo 950 terminal emulation includes a status line which

may be displayed on the 25th line by pressing the Alt T key.  The

status information displayed includes Row and Column, Insert or

Edit edit mode, Page or Line editing, Graph mode, Char or Block

mode, H.I.  (Half Intensity) mode, Protect mode, Monitor mode and

Keyboard Lock mode.

The Block or Char mode setting can be toggled while the status is

displayed by pressing the B key.  Online terminal operation is

suspended and input is enabled on the status line whenever a

blinking RIGHT ARROW appears in the first column position.  For

example, to toggle from the Block mode to the Char mode, first

display the status by entering Alt T and then press the B key.

Pressing the I key will cause the Insert or Edit mode to be

toggled.  Pressing the L key will cause the Line or Page editing

mode to be toggled.  Pressing the H key will cause the H.I.  mode

to be toggled between On and Off.  Pressing the P key will cause

the PROTECT mode setting to be toggled between On and Off.

Pressing the K key will cause the Keyboard Lock setting to be

toggled between On and Off.  Pressing the M key will toggle

Monitor mode.

TeleVideo 950                - 2 -             Terminal Emulation

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

TeleVideo 950 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 950 terminal emulation:

Keyboard 		Function

Enter			CR, LF, or CR/LF

Alt Enter		SEND

Alt Shift Enter	SHIFT SEND

Ins			CHAR INSERT

Ctrl Ins		SHIFT CHAR INSERT

Alt Ins		LINE INSERT

Alt Shift Ins		SHIFT LINE INSERT

Ctrl Esc		LOC ESC

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Ctrl Up Arrow		Shift Cursor Up

Down Arrow		Cursor Down

Ctrl Down Arrow	Shift Cursor Down

Shift PrtSc		PRINT

Alt Shift PrtSc	SHIFT PRINT

Ctrl PrtSc		Enable/Disable Printer Port

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Backspace		BS or DEL

Shift Backspace	DEL or BS

Alt A			Send Answerback Message

Alt B			Break

Alt F			FUNCT

Alt T			Display 950 Status Line

Tab			TAB

Shift Tab		BACK TAB

Home			HOME

PgUp			CLEAR SPACE

Ctrl PgUp		CLEAR ALL (Nulls)

PgDn			PAGE ERASE (Spaces)

Ctrl PgDn		PAGE ERASE (Nulls)

End			LINE ERASE (Spaces)

Ctrl End		LINE ERASE (Nulls)

Del			CHAR DELETE

Ctrl Del		SHIFT CHAR DELETE

Alt Del		LINE DELETE

Alt Shift Del		SHIFT LINE DELETE

TeleVideo 950                - 3 -             Terminal Emulation

F1	F1	Shift F1	Shift F1

F2	F2	Shift F2	Shift F2

F3	F3	Shift F3	Shift F3

F4	F4	Shift F4	Shift F4

F5	F5	Shift F5	Shift F5

F6	F6	Shift F6	Shift F6

F7	F7	Shift F7	Shift F7

F8	F8	Shift F8	Shift F8

F9	F9	Shift F9	Shift F9

F10	F10	Shift F10	Shift F10

Alt F1	F11	Alt Shift F1	Shift F11

TeleVideo 950                - 4 -             Terminal Emulation