Cermetek INFO-MATE 212A

The Cermetek INFO-MATE 212A auto-dial modem can be used with

Softerm by selecting INFO-MATE 212A as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch	  Setting	Function

1	  On		Asynchronous Operation

2	  Off		7 or 8 Bit Data with 2 Stop Bits

3	  Off		Normal CTS/DSR/DCD Operation

4	  On		Host Controlled DTR

5	  User Option	Battery Backup

6	  User Option	Exclusion Key Telephone Control

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, such as the Tandy 2000, the

CTS signal from the modem must be disconnected and CTS provided to

the serial interface through a local loopback of the RTS signal.

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.  The @ character in a phone number is not

supported.