Eicon X.25 Extended Communications File

XCM Name:              EICON.XCM

Softerm Revision:      3.00.00 or above

Base System File:      IBMPC.SYS

The Eicon extended communications file provides access to up to 4 ports.

This XCM uses the Interrupt 014H interface with the Eicon extensions.

BRINGING UP THE X.25 NETWORK -- INTRODUCTION

--------------------------------------------

Before Softerm can be used with the Eicon X.25 interface on a LAN, the X.25

network software must first be loaded and started, both on the X.25 Gateway

server and on the local PC.

To use the X.25 software, either on the server or on a workstation, a NETBIOS

(NETwork Basic Input/Output) service must first be installed.  Depending on

the network, this may be performed automatically in the process of installing

the network software (such as 3COM's 3+ or IBM's PC Network) or may require

additional commands (as is the case with Novell NetWare).  The command to

install NETBIOS for Novell NetWare is:

NETBIOS

The default number of NETBIOS sessions provided is 10.  To change the number

of sessions, include the line:

NETBIOS SESSIONS = n

in your SHELL.CFG file, where "n" is the number of sessions desired.

BRINGING UP THE X.25 NETWORK -- THE SERVER

------------------------------------------

To start the server software (the server is the machine that has the X.25

network adapter board installed), use the following commands:

NABIOS SRV server_name[ password][ /n]

X25NET START

The first command, "NABIOS", installs the network adapter BIOS which allows

communications between the X.25 link and workstations on the LAN.  The "SRV"

parameter identifies this machine as a server, and the "server_name"

parameter gives the name of this server as it will be known to all other

workstations on the LAN.  This name must be unique on the network.  The

"password" parameter is optional; it is used to restrict access to the server

rom workstations.  If it is specified, only those workstations that supply

the password in their X25NET USE command will be allowed access to the

server.

The "/n" parameter defines the maximum number of sessions that the server

will have available for use by all workstations on the network.  The "n" is

a

number from 1 through 32, inclusive, which represents the number of LAN

sessions available and has the following restrictions:

Eicon X.25 Extended Communications File                             Page 2

1.      One NETBIOS session is required for each NABIOS session.

2.      There can be no more NABIOS sessions that there are

virtual circuits available on your X.25 network subscription.

The second command, "X25NET START", installs loads the X.25 network interface

software that communicates with the interface board.  If you are using a

dedicated access facility, this command will establish the link to your X.25

service.  If your are using a non-dedicated service (i.e., the service is

accessed via a dial phone line), you must establish the connection after

issuing the "X25NET START" command.  If you are using the Eicon Technologies

Dial Network Adapter, the MODEM program supplied with the adapter is used

to make the connection.  If you are using a standalone modem connected to the

Eicon adapter, you must dial the number manually with the modem in "Voice"

mode, and switch to "Data" mode when you hear the high-pitched answer tone

from the answering modem.

You can monitor the status of the X.25 link by issuing the command:

X25NET

at the server.  This will display information about the state of the X.25

link and any applications that are using the server.

BRINGING UP THE X.25 NETWORK -- THE CLIENT

------------------------------------------

The Softerm program requires the use of the ITI (Interactive Terminal

Interface) interface supplied by Eicon. This interface is provided by the

file ITI.SYS, which must be included in the CONFIG.SYS filefor each client PC

that will use the X.25 services.  The format for the line in CONFIG.SYS is:

DEVICE = ITI.SYS

By default, ITI.SYS provides redirection service to ports COM1 through COM4.

A companion program, ITI.EXE, can be used to control the redirection of ports

COM1 and COM2.  The command line is as follows:

ITI /E=n

where "n" is either 0 to disable support for both COM1 and COM2, 1 to enable

support for COM1 and disable support for COM2, 2 to disable support for

COM1 and enable support for COM2, or 3 to enable support for COM1 and COM2.

COM3 and COM4 redirection is always enabled.  The COM port to which Softerm

sends it's output is configured in the NETCONFG program (described later).

To start the client software (a client is any of the workstations that will

use the services of the X.25 server), use the commands:

NABIOS RDR client_name[ /n]

X25NET USE server_name[ password]

Eicon X.25 Extended Communications File                             Page 3

The first command, "NABIOS",  installs the network adapter BIOS which allows

communications between the client workstation and the X.25 server. The "RDR"

parameter identifies this machine as a redirector (client), one which will

redirect data traffic normally destined for a COM port to the X.25 link.  The

"client_name" parameter gives the name of this client as it will be known to

all other

workstations on the LAN.  This name must be unique on the LAN.  The "n" is a

number from 1 through 32, inclusive, which gives the number of LAN sessions

available, and has the following restrictions:

1.      One NETBIOS session is required for each NABIOS session.

2.      There can be no more NABIOS sessions that there are

sessions that have been made available by the server

with the "NABIOS SRV" command.

The second command, "X25NET USE", establishes the connection to the server

to be used by this client.  The "server_name" corresponds to the name of the

server as given in the "NABIOS SRV" command used at the server.  The

"password" field is required only if the server was started with an

"NABIOS SRV" command that included a password definition.  If so, the

password

presented here must match the password for the server.

Once these steps have been performed, you are ready to generate and use

Softerm with the Eicon X.25 gateway on the client machine.  By placing the

preceding commands in your AUTOEXEC.BAT file, you can eliminate repeating

these steps each time you boot your machine.

CONFIGURING SOFTERM FOR USE WITH THE EICON X.25 GATEWAY

-------------------------------------------------------

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually

using

the following DOS command:

COPY IBMPC.SYS/B+EICON.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+EICON.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+EICON.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the EICON.XCM module, the

options concern the number of network communications ports, the mapped port

number and the auto-connection establishment options.

Eicon X.25 Extended Communications File                              Page 4

Up to 4 network communications ports may be defined, however, in order

for them to be usable, you must have defined at least as many sessions

in the NABIOS command as you do network communications ports.  The "Mapped to

Port #" on the configuration menu defines the COM port number to which

Interrupt 014H communications for the Eicon interface are directed.  This

number my be any number from 1-4, but the Eicon ITI software defines

whether ports 1 and 2 are enabled.  Ports 3 and 4 are always enabled.

The "Auto-Connect on Assign" option provides a mechanism by which to have

Softerm establish a connection as soon as a port assigned to the Eicon

service is selected.  If you answer this option as "Yes", Softerm will

attempt to establish a connection to the "Auto-Connect Call ID".  This call

id is in the following format:

[P][R]called_DTE[,calling_DTE][*user_data]

The "P" parameter is optional, it should be used when the calling DTE is

connected to Canada's DataPac network, and the called DTE is on another

network.  It specifies "P"riority service from DataPac.  The "R"

parameter is also optional and specifies "R"everse charging if present.

The "called_DTE" parameter is the only required parameter; it is the

network address of the DTE to which you wish to connect.  The ",calling_DTE"

parameter is optional; if present a comma must be the first character and it

specifies the network address of your DTE.  The "*user_data" parameter is

optional; if present an asterisk must be the first character and it may

include any user data information you require.  Auto-Connection is tried only

once; if it fails, you must use the Softerm Phone Book Utilities or the File

Transfer "Dial" command to establish a connection.

Each network communications port will be available on a different Softerm

COM port, mapped in the following manner:

for 1 network COM port:

Softerm's COM4 is Eicon session 0

for 2 network COM ports:

Softerm's COM4 is Eicon session 1

Softerm's COM3 is Eicon session 0

for 3 network COM ports:

Softerm's COM4 is Eicon session 2

Softerm's COM3 is Eicon session 1

Softerm's COM2 is Eicon session 0

for 4 network COM ports:

Softerm's COM4 is Eicon session 3

Softerm's COM3 is Eicon session 2

Softerm's COM2 is Eicon session 1

Softerm's COM1 is Eicon session 0

Any physical COM ports present on the PC will still be available under their

usual designations, provided that the number of network ports plus the number

of physical ports does not exceed 4.  If it does, the physical ports will be

deleted, starting at the highest numbered port, until all of the network

ports

can be accessed.

Eicon X.25 Extended Communications File                              Page 5

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The Softerm Phone Book Utilities and the File Transfer "Dial" command are

supported when using the Eicon interface.  To define a connection, enter the

connection information in the number field of the phone book entry in the

same format as described above for the "Auto-Connect Call ID".  However,

you may also include pad parameters by placing them in the "Notes/LAN

Information" field of the phone book entry.  Start the first field of the

"Notes/LAN Information" with the string "PAD>" followed by the pad

parameters separated by commas:

PAD>2:1,3:2,4:0

These pad parameters will be applied to the link approximately 5 seconds

after a connection is established; this allows some time for the remote

end to set its desired pad parameters (if any) before the local pad

parameter definitions are changed.