PROMETHEUS ProModem 1200

The PROMETHEUS ProModem 1200 modem can be used with Softerm by

selecting ProModem as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch	  Setting	  Function

1	  On		  Enable Command Recognition

2	  User Option	  RJ11/(RJ12 or RJ13) jack

3	  User Option	  Initialized by Softerm

4	  User Option	  Enable/Disable auto answer

5	  User Option	  Echo/No Echo in command state

6	  User Option	  Initialized by Softerm

7	  User Option	  Verbal/Numeric result codes

8	  Off		  Normal DTR operation

9	  Off		  Prevent modem automatically redialing

10	  Off		  Normal DCD operation

The ProModem includes true wait for dial tone capability.