ACS Extended Communications File

XCM Name:                ACS.XCM

Softerm Revision:        3.00.00 or above

Base System File:        IBMPC.SYS

The ACS extended communications file provides support for multiple

asynchronous ports controlled by either an IBM Asynchronous Communications

Server (IACS) or a SOFTERM Asynchronous Communications Server (SACS).

In order to access an IBM ACS or a SOFTERM ACS, the NetBIOS interface must

be available.  If the SOFTERM ACS version is 2.00 or above and the Local

Area Network uses Novell Netware, SOFTERM PC may access the SOFTERM ACS

using either the NetBIOS interface or the IPX/NetWare shell interface.  It

is not necessary for either the SOFTERM ACS or the client workstation to be

'logged in' to a file server.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+ACS.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+ACS.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+ACS.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.

Define the LAN type, number of communications ports, and local system name.

The local system name must be unique on the network.  SOFTERM PC will append

_COMx to the system name defined to provide each communications port with a

unique network name which is essential for NetBIOS and in an IPX/Netare

environment is used to allow the SACS monitor program to identify the name

of clients currently attached to SACS ports.

The lockup question would normally be answered as 'NO' but should be set to

'YES' if SOFTERM lockups are experienced.  It is not possible at this time

to provide a list of environments where SOFTERM may exhibit problems other

than to note that they occur in some non 100% IBM configurations.  This

option, when selected, forces SOFTERM to use different internal network

access mechanisms.

ACS Extended Communications File                                   Page 2

The "Auto-Connect on Assign" option provides a mechanism by which to have

Softerm establish a connection as soon as the port is assigned for use.

If you answer this option as "Yes", Softerm will attempt to establish a

connection to the "Auto-Connect Call ID".  This call id is in one of the

following formats:

IBM Asynchronous Communications Server:

- $Servername#Phonenumber

- $Servername"Targetname

- $Servername@CBXDatagroupid|accountid

where:

$ is used as a 'server name' introducer,

# is used as a 'phone number' introducer,

" is used as a 'target name' introducer and

| is used as a 'account id' introducer.

SOFTERM	Asynchronous Communications Server:

- $Servername[\Portname]

- $Servername[\Portname]#

- $Servername[\Portname]#Phonenumber

- $Servername[\Portname]"Targetname

where:

$ is used as a 'server name' introducer,

\ is used as an optional 'port name' introducer that

requests access to a specific group of ports configured

at the server,

# is used as a 'phone number' introducer, if no phone

number is present, SOFTERM will be connected to the

modem and allow manual presentation of a number for

dialling.  Absence of any phone number requests access

to a direct connect port,

" is used as a 'target name' introducer that is predefined

at the server in a phone book.

Auto-Connection is tried only once; if it fails, you must use the Softerm

Phone Book Utilities or the File Transfer "Dial" command to establish a

connection.

ACS Extended Communications File                                   Page 3

Any physical COM ports present on the PC will still be available under their

usual designations, provided that the number of network ports plus the number

of physical ports does not exceed 4.  If it does, the physical ports will be

deleted, starting at the highest numbered port, until all of the network ports

can be accessed.

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The ACS extended communications file supports the use of Softerm's Phone

Book and auto-dialer.  You may enter the connection information in the

number field for a phone book entry, and Softerm will establish the connection

for you.  You may also terminate a connection using the emulation status line

command "Break Connection", toggling to local mode, or using the Phone Book

"Hangup" command.

To establish a connection through the network, invoke the Softerm dialing

directory function from Softerm's Online mode of execution by pressing Alt-2

followed by function key 2.  Select the "Dial Number" option, and press

return.  Press return again to position the cursor at the number field.

This is where to enter the information for establishing the connection.