Anchor Automation SIGNALMAN MARK XII

The Anchor Automation SIGNALMAN MARK XII auto-dial modem can be

used with Softerm by selecting SIGNALMAN MARK XII as the modem

dialer type.

The MARK XII does not allow local control of the DTR signal.  This

means that a call cannot be disconnected by dropping DTR by

selecting local mode on the Softerm status line.  To hangup a

connection, use the Phone Book Utilities Hangup option or enter

the escape to command mode sequence +++ followed by a hangup

command AT H in the terminal mode.

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.  The @ character in a phone number is not

supported.  The use of the @ character in a phone number will

cause a blind 4 second wait.