Rixon R212A Intelligent Modem

The Rixon R212A Intelligent Modem can be used with Softerm by

selecting Rixon R212A as the modem dialer type.  The modem also

includes a Hayes Smartmodem compatible mode which can be used with

the Hayes Smartmodem dialer type.

The Softerm driver is designed to work with the default Rixon

options in the Rixon mode.  Since carrier is constantly present in

the Rixon mode, PAUSE or XMIT:WAIT commands must be used when

using a DIAL command in a macro.  However, the Rixon modem option

"CTS ON" can be set to "N", and a modified cable can be used to

allow Softerm to wait for carrier and detect disconnects properly.

To modify the cable, disconnect CTS (Pin 5) from the modem and

jumper RTS (Pin 4) to CTS so that characters can still be

transmitted to the modem.