Allen-Bradley Personal LAN/1 Port (PLP) Extended Communications File

XCM Name:              AB-PLP.XCM

Softerm Revision:      3.00.00 or above

Base System File:      IBMPC.SYS

The Allen-Bradley PLP extended communications file provides access to one

port via the Personal LAN/1 Port interface.  This XCM uses the Interrupt 014H

interface.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+AB-PLP.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+AB-PLP.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+AB-PLP.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the AB-PLP.XCM module, the

only option is the COM port number to which communications for the Allen-

Bradley PLP will be directed.  This is defined as the "Mapped to Port #"

on the configuration menu, and, for the last version of the Allen-Bradley

PLP tested, should be set to 1.  However, if your PLP software allows it, the

number may be changed to 2, 3 or 4.  This defines the COM port number that

will be used to communicate with the Interrupt 014H software provided by

Allen-Bradley.  As far as Softerm PC is concerned, it will always know the

Allen-Bradley PLP port as COM4.  This is the port name that you should pick

when you wish to use the PLP connection.  Any physical COM ports present on

the PC will still be available under their usual designations, provided that

the number of network ports plus the number of physical ports does not exceed

4.  If it does, the physical ports will be deleted, starting at the highest

numbered port, until all of the network ports can be accessed.

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

Allen-Bradley PLP Extended Communications File                         Page 2

The Allen-Bradley PLP extended communications file supports the use of

Softerm's Phone Book and auto-dialer.  You may enter the connection

information in the number field for a phone book entry, and Softerm will

establish the connection for you.  You may also terminate a connection using

the emulation status line command "Break Connection", toggling to local mode,

or using the Phone Book "Hangup" command.  An example of a phone book entry

might be:

If you would normally enter:

C 128,0,0,94,4

to establish a connection to port 4 on a VistaLAN/1 NIU at address 94, with

the Internet Protocol address of 128,0,0; you can instead enter:

128,0,0,94,4

for the number of an entry in the phone book.  You may then select that entry

to dial whenever you wish to establish the connection.