U.S.  Robotics Auto-Dial 212A

The U.S.  Robotics Auto-Dial 212A modem can be used with Softerm

by selecting USR AD 212A as the modem dialer type.  For proper

operation, the configuration switches must be set as follows:

Switch		Setting

RNG		Use as specified in manual

8HI		On

BAL		Use as specified in manual

DTR		Off

Pin 4 may have to be looped back to Pin 5 on the serial I/O

interface, depending on the revision of the modem.

U.S.  Robotics Password 300/1200

The U.S.  Robotics Password modem can be used with Softerm by

selecting Hayes Smartmodem as the modem dialer type.  For proper

operation, set all configuration switches to the Off position.