Ven-Tel MD212 Plus Auto-Dial Modem

The Ven-Tel MD212 Plus auto-dial modem can be used with Softerm by

selecting Ven-Tel MD212 Plus as the modem dialer type.

The MD212 Plus does not allow software control of the type of

dialing used.  Switch H1 on the standalone MD212 Plus if UP will

cause pulse dialing and if DOWN will cause tone dialing.  On the

rackmount version, Switch S7-1 will cause pulse dialing if ON and

tone dialing if OFF.